Overview

Data Dictionary for ""

"Accident" package

D2Payload/PayloadPublication/Situation/Classes/TrafficElement/Accident

"Accident" package classes

Table A.1— Classes of the "Accident" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| Accident | Accident | Accidents are events where one or more vehicles are involved in collisions or in leaving the roadway. These include collisions between vehicles or with other road users or obstacles. | D2Class | no |

Associations of "Accident" package

Table A.2— Associations of the "Accident" package

| Class name | Association end | Designation | Definition | Multiplicity | Target |
| --- | --- | --- | --- | --- | --- |
| Accident | vehicleInvolved | Vehicle involved | The vehicle involved in the accident. | 0..* | Vehicle |

"Accident" package attributes

Table A.3— Attributes of the "Accident" package

| Class name | Attribute name | Designation | Definition | Multiplicity | Type |
| --- | --- | --- | --- | --- | --- |
| Accident | accidentType | Accident type | A characterization of the nature of the accident. | 1..* | AccidentTypeEnum |

"AlertC" package

D2Payload/LocationReferencing/LocationReference/AlertC

"AlertC" package classes

Table A.4— Classes of the "AlertC" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| AlertCDirection | ALERT-C direction | The direction of traffic flow along the road to which the information relates. | D2Class | no |
| AlertCLinear | ALERT-C linear | A linear section along a road defined between two points on the road by reference to a pre-defined ALERT-C location table. | D2Class | yes |
| AlertCLocation | ALERT-C location | Identification of a specific point, linear or area location in an ALERT-C location table. | D2Class | no |
| AlertCMethod2PrimaryPointLocation | ALERT-C method2 primary point location | The point (called Primary point) which is either a single point or at the downstream end of a linear road section. The point is specified by a reference to a point in a pre-defined ALERT-C location table. | D2Class | no |
| AlertCMethod2SecondaryPointLocation | ALERT-C method2 secondary point location | The point (called Secondary point) which is at the upstream end of a linear road section. The point is specified by a reference to a point in a pre-defined ALERT-C location table. | D2Class | no |
| AlertCMethod4PrimaryPointLocation | ALERT-C method4 primary point location | The point (called Primary point) which is either a single point or at the downstream end of a linear road section. The point is specified by a reference to a point in a pre-defined ALERT-C location table plus a non-negative offset distance. | D2Class | no |
| AlertCMethod4SecondaryPointLocation | ALERT-C method4 secondary point location | The point (called Secondary point) which is at the upstream end of a linear road section. The point is specified by a reference to a point in a pre-defined Alert-C location table plus a non-negative offset distance. | D2Class | no |
| AlertCPoint | ALERT-C point | A single point on the road network defined by reference to a pre-defined ALERT-C location table and which has an associated direction of traffic flow. | D2Class | yes |

Associations of "AlertC" package

Table A.5— Associations of the "AlertC" package

| Class name | Association end | Designation | Definition | Multiplicity | Target |
| --- | --- | --- | --- | --- | --- |
| AlertCMethod2PrimaryPointLocation | alertCLocation | ALERT-C location |  | 1..1 | AlertCLocation |
| AlertCMethod2SecondaryPointLocation | alertCLocation | ALERT-C location |  | 1..1 | AlertCLocation |
| AlertCMethod4PrimaryPointLocation | alertCLocation | ALERT-C location |  | 1..1 | AlertCLocation |
|  | offsetDistance | Offset distance |  | 1..1 | OffsetDistance |
| AlertCMethod4SecondaryPointLocation | alertCLocation | ALERT-C location |  | 1..1 | AlertCLocation |
|  | offsetDistance | Offset distance |  | 1..1 | OffsetDistance |

"AlertC" package attributes

Table A.6— Attributes of the "AlertC" package

| Class name | Attribute name | Designation | Definition | Multiplicity | Type |
| --- | --- | --- | --- | --- | --- |
| AlertCDirection | alertCAffectedDirection | ALERT-C affected direction | The direction(s) of traffic flow to which the situation, traffic data or information is related. | 1..1 | LinearDirectionEnum |
|  | alertCDirectionCoded | ALERT-C direction coded | Direction of navigation with respect to secondary to primary location (RDS direction) | 1..1 | AlertCDirectionEnum |
|  | alertCDirectionNamed | ALERT-C direction named | ALERT-C name of a direction e.g. Brussels -> Lille. | 0..1 | MultilingualString |
| AlertCLinear | alertCLocationCountryCode | ALERT-C location country code | ALERT-C country code as defined in IEC 62106. | 1..1 | String |
|  | alertCLocationTableNumber | ALERT-C location table number | Number allocated to an ALERT-C table in a country. Ref. EN ISO 14819-3 for the allocation of a location table number. | 1..1 | String |
|  | alertCLocationTableVersion | ALERT-C location table version | Version number associated with an ALERT-C table reference. | 1..1 | String |
| AlertCLocation | alertCLocationName | ALERT-C location name | Name of ALERT-C location. | 0..1 | MultilingualString |
|  | specificLocation | Specific location | Unique code within the ALERT-C location table which identifies the specific point, linear or area location. | 1..1 | AlertCLocationCode |
| AlertCPoint | alertCLocationCountryCode | ALERT-C location country code | ALERT-C country code as defined IEC 62106. | 1..1 | String |
|  | alertCLocationTableNumber | ALERT-C location table number | Number allocated to an ALERT-C table in a country. Ref. EN ISO 14819-3 for the allocation of a location table number. | 1..1 | String |
|  | alertCLocationTableVersion | ALERT-C location table version | Version number associated with an ALERT-C table reference. | 1..1 | String |

"AlertCArea" package

D2Payload/LocationReferencing/LocationReference/AlertC/AlertCArea

"AlertCArea" package classes

Table A.7— Classes of the "AlertCArea" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| AlertCArea | ALERT-C area | An area defined by reference to a predefined ALERT-C location table. | D2Class | no |

Associations of "AlertCArea" package

Table A.8— Associations of the "AlertCArea" package

| Class name | Association end | Designation | Definition | Multiplicity | Target |
| --- | --- | --- | --- | --- | --- |
| AlertCArea | areaLocation | Area location | Area location defined by a specific Alert-C location. | 1..1 | AlertCLocation |

"AlertCArea" package attributes

Table A.9— Attributes of the "AlertCArea" package

| Class name | Attribute name | Designation | Definition | Multiplicity | Type |
| --- | --- | --- | --- | --- | --- |
| AlertCArea | alertCLocationCountryCode | ALERT-C location country code | ALERT-C country code as defined in IEC 62106. | 1..1 | String |
|  | alertCLocationTableNumber | ALERT-C location table number | Number allocated to an ALERT-C table in a country. Ref. EN ISO 14819-3 for the allocation of a location table number. | 1..1 | String |
|  | alertCLocationTableVersion | ALERT-C location table version | Version number associated with an ALERT-C table reference. | 1..1 | String |

"AlertCLinearByCode" package

D2Payload/LocationReferencing/LocationReference/AlertC/AlertCLinearByCode

"AlertCLinearByCode" package classes

Table A.10— Classes of the "AlertCLinearByCode" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| AlertCLinearByCode | ALERT-C linear by code | A linear section along a road defined by reference to a linear section in a pre-defined ALERT-C location table. | D2Class | no |

Associations of "AlertCLinearByCode" package

Table A.11— Associations of the "AlertCLinearByCode" package

| Class name | Association end | Designation | Definition | Multiplicity | Target |
| --- | --- | --- | --- | --- | --- |
| AlertCLinearByCode | alertCDirection | ALERT-C direction |  | 1..1 | AlertCDirection |
|  | locationCodeForLinearLocation | Location code for linear location | Linear location defined by a specific Alert-C location. | 1..1 | AlertCLocation |

"AlertCLinearByCode" package attributes

There are no defined attributes in the "AlertCLinearByCode" package.

"AlertCMethod2Linear" package

D2Payload/LocationReferencing/LocationReference/AlertC/AlertCMethod2Linear

"AlertCMethod2Linear" package classes

Table A.12— Classes of the "AlertCMethod2Linear" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| AlertCMethod2Linear | ALERT-C method2 linear | A linear section along a road between two points, primary and secondary, which are pre-defined in an ALERT-C location table. Direction is FROM the secondary point TO the primary point, i.e. the primary point is downstream of the secondary point. | D2Class | no |

Associations of "AlertCMethod2Linear" package

Table A.13— Associations of the "AlertCMethod2Linear" package

| Class name | Association end | Designation | Definition | Multiplicity | Target |
| --- | --- | --- | --- | --- | --- |
| AlertCMethod2Linear | alertCDirection | ALERT-C direction |  | 1..1 | AlertCDirection |
|  | alertCMethod2PrimaryPointLocation | ALERT-C method2 primary point location |  | 1..1 | AlertCMethod2PrimaryPointLocation |
|  | alertCMethod2SecondaryPointLocation | ALERT-C method2 secondary point location |  | 1..1 | AlertCMethod2SecondaryPointLocation |

"AlertCMethod2Linear" package attributes

There are no defined attributes in the "AlertCMethod2Linear" package.

"AlertCMethod2Point" package

D2Payload/LocationReferencing/LocationReference/AlertC/AlertCMethod2Point

"AlertCMethod2Point" package classes

Table A.14— Classes of the "AlertCMethod2Point" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| AlertCMethod2Point | ALERT-C method2 point | A single point on the road network defined by reference to a point in a pre-defined ALERT-C location table and which has an associated direction of traffic flow. | D2Class | no |

Associations of "AlertCMethod2Point" package

Table A.15— Associations of the "AlertCMethod2Point" package

| Class name | Association end | Designation | Definition | Multiplicity | Target |
| --- | --- | --- | --- | --- | --- |
| AlertCMethod2Point | alertCDirection | ALERT-C direction |  | 1..1 | AlertCDirection |
|  | alertCMethod2PrimaryPointLocation | ALERT-C method2 primary point location |  | 1..1 | AlertCMethod2PrimaryPointLocation |

"AlertCMethod2Point" package attributes

There are no defined attributes in the "AlertCMethod2Point" package.

"AlertCMethod4Linear" package

D2Payload/LocationReferencing/LocationReference/AlertC/AlertCMethod4Linear

"AlertCMethod4Linear" package classes

Table A.16— Classes of the "AlertCMethod4Linear" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| AlertCMethod4Linear | ALERT-C method4 linear | A linear section along a road between two points, primary and secondary, which are pre-defined ALERT-C locations plus offset distance. Direction is FROM the secondary point TO the primary point, i.e. the primary point is downstream of the secondary point. | D2Class | no |

Associations of "AlertCMethod4Linear" package

Table A.17— Associations of the "AlertCMethod4Linear" package

| Class name | Association end | Designation | Definition | Multiplicity | Target |
| --- | --- | --- | --- | --- | --- |
| AlertCMethod4Linear | alertCDirection | ALERT-C direction |  | 1..1 | AlertCDirection |
|  | alertCMethod4PrimaryPointLocation | ALERT-C method4 primary point location |  | 1..1 | AlertCMethod4PrimaryPointLocation |
|  | alertCMethod4SecondaryPointLocation | ALERT-C method4 secondary point location |  | 1..1 | AlertCMethod4SecondaryPointLocation |

"AlertCMethod4Linear" package attributes

There are no defined attributes in the "AlertCMethod4Linear" package.

"AlertCMethod4Point" package

D2Payload/LocationReferencing/LocationReference/AlertC/AlertCMethod4Point

"AlertCMethod4Point" package classes

Table A.18— Classes of the "AlertCMethod4Point" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| AlertCMethod4Point | ALERT-C method4 point | A single point on the road network defined by reference to a point in a pre-defined ALERT-C location table plus an offset distance and which has an associated direction of traffic flow. | D2Class | no |

Associations of "AlertCMethod4Point" package

Table A.19— Associations of the "AlertCMethod4Point" package

| Class name | Association end | Designation | Definition | Multiplicity | Target |
| --- | --- | --- | --- | --- | --- |
| AlertCMethod4Point | alertCDirection | ALERT-C direction |  | 1..1 | AlertCDirection |
|  | alertCMethod4PrimaryPointLocation | ALERT-C method4 primary point location |  | 1..1 | AlertCMethod4PrimaryPointLocation |

"AlertCMethod4Point" package attributes

There are no defined attributes in the "AlertCMethod4Point" package.

"Classes" package

D2Payload/Common/Classes

"Classes" package classes

Table A.20— Classes of the "Classes" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| HeaderInformation | Header information | Management information relating to the data contained within a publication. | D2Class | no |
| InternationalIdentifier | International identifier | An identifier/name whose range is specific to the particular country. | D2Class | no |
| PayloadPublication | Payload publication | A payload publication of traffic related information or associated management information created at a specific point in time that can be exchanged via a DATEX II interface. | D2ModelRoot | yes |

Associations of "Classes" package

Table A.21— Associations of the "Classes" package

| Class name | Association end | Designation | Definition | Multiplicity | Target |
| --- | --- | --- | --- | --- | --- |
| PayloadPublication | publicationCreator | Publication creator |  | 1..1 | InternationalIdentifier |

"Classes" package attributes

Table A.22— Attributes of the "Classes" package

| Class name | Attribute name | Designation | Definition | Multiplicity | Type |
| --- | --- | --- | --- | --- | --- |
| HeaderInformation | informationStatus | Information status | The status of the related information (real, test, exercise ....). | 1..1 | InformationStatusEnum |
| InternationalIdentifier | country | Country | EN ISO 3166-1 two-character country code. | 1..1 | CountryCode |
|  | nationalIdentifier | National identifier | Identifier or name unique within the specified country. | 1..1 | String |
| PayloadPublication | defaultLanguage | Default language | The default language used throughout the payload publication. | 1..1 | Language |
|  | publicationTime | Publication time | Date/time at which the payload publication was created. | 1..1 | DateTime |

"Classes" package

D2Payload/PayloadPublication/Situation/Classes

"Classes" package classes

Table A.23— Classes of the "Classes" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| Situation | Situation | An identifiable instance of a traffic/travel situation comprising one or more traffic/travel circumstances which are linked by one or more causal relationships. Each traffic/travel circumstance is represented by a Situation Record. | D2Class | no |
| SituationRecord | Situation record | An identifiable versioned instance of a single record/element within a situation. | D2Class | yes |

Associations of "Classes" package

Table A.24— Associations of the "Classes" package

| Class name | Association end | Designation | Definition | Multiplicity | Target |
| --- | --- | --- | --- | --- | --- |
| Situation | headerInformation | Header information |  | 1..1 | HeaderInformation |
|  | situationRecord | Situation record | Details of the Situation | 1..* | SituationRecord |
| SituationRecord | validity | Validity |  | 1..1 | Validity |
|  | locationReference | Location reference |  | 1..1 | LocationReference |

"Classes" package attributes

Table A.25— Attributes of the "Classes" package

| Class name | Attribute name | Designation | Definition | Multiplicity | Type |
| --- | --- | --- | --- | --- | --- |
| SituationRecord | probabilityOfOccurrence | Probability of occurrence | An assessment of the degree of likelihood that the reported event will occur. | 1..1 | ProbabilityOfOccurrenceEnum |
|  | safetyRelatedMessage | Safety related message | Indicates, whether this SituationRecord specifies a safety related message according to Commission Delegated Regulation (EU) No 886/2013. | 0..1 | Boolean |
|  | situationRecordCreationTime | Situation record creation time | The date/time that the SituationRecord object (the first version of the record) was created by the original supplier. | 1..1 | DateTime |
|  | situationRecordVersionTime | Situation record version time | The date/time that this current version of the SituationRecord within the situation was written into the database of the supplier which is involved in the data exchange. Identity and version of record are defined by the class stereotype implementation. | 1..1 | DateTime |

"Conditions" package

D2Payload/PayloadPublication/Situation/Classes/TrafficElement/Conditions

"Conditions" package classes

Table A.26— Classes of the "Conditions" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| Conditions | Conditions | Any conditions which have the potential to degrade normal driving conditions. | D2Class | no |
| NonWeatherRelatedRoadConditions | Non weather related road conditions | Road surface conditions that are not related to the weather but which may affect driving conditions. | D2Class | no |
| RoadSurfaceConditions | Road surface conditions | Conditions of the road surface which may affect driving conditions. These may be related to the weather (e.g. ice, snow etc.) or to other conditions (e.g. oil, mud, leaves etc. on the road) | D2Class | yes |
| WeatherRelatedRoadConditions | Weather related road conditions | Road surface conditions that are related to the weather which may affect the driving conditions, such as ice, snow or water. | D2Class | no |

Associations of "Conditions" package

There are no defined associations in the "Conditions" package.

"Conditions" package attributes

Table A.27— Attributes of the "Conditions" package

| Class name | Attribute name | Designation | Definition | Multiplicity | Type |
| --- | --- | --- | --- | --- | --- |
| NonWeatherRelatedRoadConditions | nonWeatherRelatedRoadConditionType | Non weather related road condition type | The type of road conditions which are not related to the weather. | 1..* | NonWeatherRelatedRoadConditionTypeEnum |
| WeatherRelatedRoadConditions | weatherRelatedRoadConditionType | Weather related road condition type | The type of road surface condition that is related to the weather which is affecting the driving conditions. | 1..* | WeatherRelatedRoadConditionTypeEnum |

"DataValue" package

D2Payload/Common/Classes/DataValue

"DataValue" package classes

Table A.28— Classes of the "DataValue" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| DataValue | Data value | A data value of something that can be measured or calculated. Any provided meta-data values specified in the attributes override any specified generic characteristics such as defined for a specific measurement in the MeasurementSiteTable. | D2Class | yes |
| SpeedValue | Speed value | A measured or calculated value of speed. | D2Class | no |
| VehicleFlowValue | Vehicle flow value | A measured or calculated value of the flow rate of vehicles. | D2Class | no |

Associations of "DataValue" package

There are no defined associations in the "DataValue" package.

"DataValue" package attributes

Table A.29— Attributes of the "DataValue" package

| Class name | Attribute name | Designation | Definition | Multiplicity | Type |
| --- | --- | --- | --- | --- | --- |
| DataValue | accuracy | Accuracy | The extent to which the value is expected to be free from error, measured as a percentage of the data value. 100% means fully accurate. | 0..1 | Percentage |
|  | computationalMethod | Computational method | Method of computation which has been used to compute this data value. | 0..1 | ComputationMethodEnum |
|  | dataError | Data error | Indication of whether the value is deemed to be erroneous by the supplier (true = erroneous). If not present, the data value is assumed to be ok. This may be used when automatic fault detection information relating to sensors is available. | 0..1 | Boolean |
|  | numberOfIncompleteInputs | Number of incomplete inputs | The number of inputs detected but not completed during the sampling or measurement period; e.g. vehicles detected entering but not exiting the detection zone. | 0..1 | NonNegativeInteger |
|  | numberOfInputValuesUsed | Number of input values used | The number of input values used in the sampling or measurement period to determine the data value. | 0..1 | NonNegativeInteger |
|  | reasonForDataError | Reason for data error | The reason why the value is deemed to be erroneous by the supplier. | 0..1 | MultilingualString |
|  | smoothingFactor | Smoothing factor | Coefficient required when a moving average is computed to give specific weights to the former average and the new data. A typical formula is, F being the smoothing factor: New average = (old average) F + (new data) (1 - F). | 0..1 | Float |
|  | standardDeviation | Standard deviation | The standard deviation of the sample of input values from which this value was derived, measured in the units of the data value. | 0..1 | Float |
|  | supplierCalculatedDataQuality | Supplier calculated data quality | A measure of data quality assigned to the value by the supplier. 100% equates to ideal/perfect quality. The method of calculation is supplier specific and needs to be agreed between supplier and client. | 0..1 | Percentage |
| SpeedValue | speed | Speed | A value of speed expressed in kilometres per hour. | 1..1 | KilometresPerHour |
| VehicleFlowValue | vehicleFlowRate | Vehicle flow rate | A value of vehicle flow rate expressed in vehicles per hour. | 1..1 | VehiclesPerHour |

"Gml" package

D2Payload/LocationReferencing/LocationReference/Gml

"Gml" package classes

Table A.30— Classes of the "Gml" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| GmlLinearRing | Gml linear ring | Closed line string not self-intersecting (i.e. having as last point the first point) | D2Class | no |
| GmlLineString | Gml line string | Line string based on GML (EN ISO 19136) definition: a curve defined by a series of two or more coordinate tuples. Unlike GML may be self-intersecting. If srsName attribute is not present, posList is assumed to use "ETRS89-LatLonh" reference system. | D2Class | no |
| GmlMultiPolygon | Gml multi polygon | An area defined by a set of polygons acording to GML (EN ISO 19136). | D2Class | no |
| GmlPolygon | Gml polygon | Planar surface defined by 1 exterior boundary and 0 or more interior boundaries | D2Class | no |

Associations of "Gml" package

Table A.31— Associations of the "Gml" package

| Class name | Association end | Designation | Definition | Multiplicity | Target |
| --- | --- | --- | --- | --- | --- |
| GmlMultiPolygon | gmlPolygon | Gml polygon |  | 1..* | GmlPolygon |
| GmlPolygon | exterior | Exterior | A boundary of a polygonal surface consisting of a ring i.e. in the normal 2D case, a closed polygonal line distinguished as exterior. Such a polygonal line has at least 4 pairs of coordinates | 1..1 | GmlLinearRing |
|  | interior | Interior | A boundary of internal patches of a polygonal surface consisting of a ring feature | 0..* | GmlLinearRing |

"Gml" package attributes

Table A.32— Attributes of the "Gml" package

| Class name | Attribute name | Designation | Definition | Multiplicity | Type |
| --- | --- | --- | --- | --- | --- |
| GmlLineString | posList | Pos list | List of coordinate Tuples define the geometry of this GmlLineString. There must be at least 2 Tuples of coordinates. | 1..1 | GmlPosList |
|  | srsDimension | Srs dimension | Provides the size of the tuple of coordinates of each point. This number is 2 or 3. By default when omitted the dimension shall be interpreted as 2. | 0..1 | NonNegativeInteger |
|  | srsName | Srs name | Specifies the Coordinate Reference System (CRS) used to interpret the coordinates in this GmlLineString | 0..1 | String |
| GmlMultiPolygon | gmlAreaName | Gml area name | Name of the multi-polygon area | 0..1 | MultilingualString |

"LinearWithinLinearElement" package

D2Payload/LocationReferencing/LocationReference/LinearReferencing/LinearWithinLinearElement

"LinearWithinLinearElement" package classes

Table A.33— Classes of the "LinearWithinLinearElement" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| LinearWithinLinearElement | Linear within linear element | A linear section along a linear element where the linear element is either a part of or the whole of a linear object (i.e. a road), consistent with ISO 19148 definitions. | D2Class | no |

Associations of "LinearWithinLinearElement" package

Table A.34— Associations of the "LinearWithinLinearElement" package

| Class name | Association end | Designation | Definition | Multiplicity | Target |
| --- | --- | --- | --- | --- | --- |
| LinearWithinLinearElement | linearElement | Linear element |  | 1..1 | LinearElement |
|  | fromPoint | From point | A point on the linear element that defines the start node of the linear section. | 1..1 | DistanceAlongLinearElement |
|  | toPoint | To point | A point on the linear element that defines the end node of the linear section. | 1..1 | DistanceAlongLinearElement |

"LinearWithinLinearElement" package attributes

Table A.35— Attributes of the "LinearWithinLinearElement" package

| Class name | Attribute name | Designation | Definition | Multiplicity | Type |
| --- | --- | --- | --- | --- | --- |
| LinearWithinLinearElement | administrativeAreaOfLinearSection | Administrative area of linear section | Identification of the road administration area which contains the specified linear section. | 0..1 | MultilingualString |
|  | directionOnLinearSection | Direction on linear section | The direction of traffic flow on the linear section in terms of general destination direction. | 0..1 | DirectionEnum |
|  | directionRelativeOnLinearSection | Direction relative on linear section | The direction of traffic flow on the linear section relative to the direction in which the linear element is defined. | 0..1 | LinearDirectionEnum |
|  | heightGradeOfLinearSection | Height grade of linear section | Identification of whether the linear section that is part of the linear element is at, above or below the normal elevation of a linear element of that type (e.g. road or road section) at that location, typically used to indicate "grade" separation. | 0..1 | HeightGradeEnum |

"LocationReference" package

D2Payload/LocationReferencing/LocationReference

"LocationReference" package classes

Table A.36— Classes of the "LocationReference" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| AreaLocation | Area location | Location representing a geographic or geometric defined area which may be qualified by height information to provide additional geospatial discrimination (e.g. for snow in an area but only above a certain altitude). | D2Class | no |
| Itinerary | Itinerary | Multiple (i.e. more than one) physically separate locations arranged as an ordered set that defines an itinerary or route. | D2Class | yes |
| ItineraryByIndexedLocations | Itinerary by indexed locations | Multiple physically separate locations arranged as an ordered set that defines an itinerary or route. The index qualifier indicates the order. | D2Class | no |
| LinearLocation | Linear location | Location representing a linear section with optional directionality defined between two points. | D2Class | no |
| Location | Location | The specification of a location either on a network (as a point or a linear location) or as an area. This may be provided in one or more referencing systems. | D2Class | yes |
| LocationReference | Location reference | Represents one or more physically separate locations. Multiple locations may be related, as in an itinerary or route, or may be unrelated. One LocationReference should not use multiple Location objects to represent the same physical location. | D2Class | yes |
| NetworkLocation | Network location | The specification of a location on a network (as a point or a linear location). | D2Class | yes |
| OffsetDistance | Offset distance | The non-negative offset distance from the ALERT-C referenced point to the actual point. | D2Class | no |
| PointLocation | Point location | Location representing a single geospatial point. | D2Class | no |
| SingleRoadLinearLocation | Single road linear location | Location representing a linear section along a single road with optional directionality defined between two points on the same road. No matter the kind of linear reference it uses, the constraint of using only a single road must be preserved. | D2Class | no |

Associations of "LocationReference" package

Table A.37— Associations of the "LocationReference" package

| Class name | Association end | Designation | Definition | Multiplicity | Target |
| --- | --- | --- | --- | --- | --- |
| AreaLocation | alertCArea | ALERT-C area |  | 0..* | AlertCArea |
|  | tpegAreaLocation | TPEG area location |  | 0..1 | TpegAreaLocation |
|  | namedArea | Named area |  | 0..1 | NamedArea |
|  | gmlMultiPolygon | Gml multi polygon |  | 0..1 | GmlMultiPolygon |
|  | openlrAreaLocationReference | Openlr area location reference |  | 0..1 | OpenlrAreaLocationReference |
| LinearLocation | openlrLinear | Openlr linear |  | 0..1 | OpenlrLinear |
|  | gmlLineString | Gml line string |  | 0..1 | GmlLineString |
|  | secondarySupplementaryDescription | Secondary supplementary description | Supplementary description that applies to the secondary end of the linear location. Use when properties change along the Linear. For a one-way linear the secondary end should be the destination end. | 0..1 | SupplementaryPositionalDescription |
| PointLocation | pointByCoordinates | Point by coordinates |  | 0..1 | PointByCoordinates |
|  | pointAlongLinearElement | Point along linear element |  | 0..* | PointAlongLinearElement |
|  | alertCPoint | ALERT-C point | The point location expressed using AlertC. Multiple instances of AlertCPoint shall represent the same real-world geographic feature. | 0..* | AlertCPoint |
|  | tpegPointLocation | TPEG point location |  | 0..1 | TpegPointLocation |
|  | openlrPointLocationReference | Openlr point location reference |  | 0..1 | OpenlrPointLocationReference |
| SingleRoadLinearLocation | tpegLinearLocation | TPEG linear location |  | 0..1 | TpegLinearLocation |
|  | alertCLinear | ALERT-C linear | The linear location expressed using AlertC. Multiple instances of AlertCLinear shall represent the same real-world geographic feature. | 0..* | AlertCLinear |
|  | linearWithinLinearElement | Linear within linear element |  | 0..* | LinearWithinLinearElement |
| ItineraryByIndexedLocations | locationContainedInItinerary | Location contained in itinerary | A location contained in an itinerary (i.e. an ordered set of locations defining a route or itinerary). | 1..1 | Location |
| NetworkLocation | supplementaryPositionalDescription | Supplementary positional description |  | 0..1 | SupplementaryPositionalDescription |
| Location | coordinatesForDisplay | Coordinates for display | Coordinates that may be used by clients for visual display on user interfaces. | 0..1 | PointCoordinates |

"LocationReference" package attributes

Table A.38— Attributes of the "LocationReference" package

| Class name | Attribute name | Designation | Definition | Multiplicity | Type |
| --- | --- | --- | --- | --- | --- |
| AreaLocation | areasAtWhichApplicable | Areas at which applicable | Places, in generic terms, at which the corresponding information applies. | 0..1 | AreaPlacesEnum |
| OffsetDistance | offsetDistance | Offset distance | The non-negative offset distance from the ALERT-C referenced point to the actual point. The ALERT-C locations in the primary and secondary locations must always encompass the linear section being specified, thus offset distance is towards the other point. | 1..1 | MetresAsNonNegativeInteger |

"NamedArea" package

D2Payload/LocationReferencing/LocationReference/NamedArea

"NamedArea" package classes

Table A.39— Classes of the "NamedArea" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| IsoNamedArea | Iso named area | The ISO 3166-2 representation for the named area. | D2Class | no |
| NamedArea | Named area | An area defined by a name and/or in terms of known boundaries, such as country or county boundaries or allocated control area of particular authority. The attributes do not form a union; instead, the smallest intersection forms the resulting area. | D2Class | no |
| NutsNamedArea | Nuts named area | The NUTS-Code representation for the named area (Nomenclature of territorial units for statistics) or its LAU code representation (Local Administrative Unit). | D2Class | no |

Associations of "NamedArea" package

There are no defined associations in the "NamedArea" package.

"NamedArea" package attributes

Table A.40— Attributes of the "NamedArea" package

| Class name | Attribute name | Designation | Definition | Multiplicity | Type |
| --- | --- | --- | --- | --- | --- |
| IsoNamedArea | subdivisionCode | Subdivision code | The ISO 3166-2 subdivision code for the named area. | 1..1 | SubdivisionCode |
|  | subdivisionType | Subdivision type | The ISO 3166-2 subdivison type for the named area. | 1..1 | SubdivisionTypeEnum |
| NamedArea | areaName | Area name | The name of the area. | 1..1 | MultilingualString |
|  | country | Country | EN ISO 3166-1 two-character country code. | 0..1 | CountryCode |
|  | namedAreaType | Named area type | The type of the area. | 0..1 | NamedAreaTypeEnum |
| NutsNamedArea | nutsCode | Nuts code | The NUTS code for the named area. | 1..1 | NutsCode |
|  | nutsCodeType | Nuts code type | The NUTS code type for the named area. | 1..1 | NutsCodeTypeEnum |

"Obstruction" package

D2Payload/PayloadPublication/Situation/Classes/TrafficElement/Obstruction

"Obstruction" package classes

Table A.41— Classes of the "Obstruction" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| AnimalPresenceObstruction | Animal presence obstruction | An obstruction on the road resulting from the presence of animals. | D2Class | no |
| EnvironmentalObstruction | Environmental obstruction | An obstruction on the road resulting from an environmental cause. | D2Class | no |
| GeneralObstruction | General obstruction | Any stationary or moving obstacle of a physical nature, other than of an animal, vehicle, environmental, or damaged equipment nature. | D2Class | no |
| Obstruction | Obstruction | Any stationary or moving obstacle of a physical nature (e.g. obstacles or vehicles from an earlier accident, shed loads on carriageway, rock fall, abnormal or dangerous loads, or animals etc.) which could disrupt or endanger traffic. | D2Class | yes |
| VehicleObstruction | Vehicle obstruction | An obstruction on the road caused by one or more vehicles. | D2Class | no |

Associations of "Obstruction" package

There are no defined associations in the "Obstruction" package.

"Obstruction" package attributes

Table A.42— Attributes of the "Obstruction" package

| Class name | Attribute name | Designation | Definition | Multiplicity | Type |
| --- | --- | --- | --- | --- | --- |
| AnimalPresenceObstruction | animalPresenceType | Animal presence type | Indicates the nature of animals present on or near the roadway. | 1..1 | AnimalPresenceTypeEnum |
| EnvironmentalObstruction | environmentalObstructionType | Environmental obstruction type | Characterization of an obstruction on the road resulting from an environmental cause. | 1..1 | EnvironmentalObstructionTypeEnum |
| GeneralObstruction | obstructionType | Obstruction type | Characterization of the type of general obstruction. | 1..* | ObstructionTypeEnum |
| VehicleObstruction | involvedVehicleType | Involved vehicle type | Type of vehicle involved | 0..1 | VehicleTypeEnum |
|  | vehicleObstructionType | Vehicle obstruction type | Characterization of an obstruction on the road caused by one or more vehicles. | 1..1 | VehicleObstructionTypeEnum |

"OpenLR" package

D2Payload/LocationReferencing/LocationReference/OpenLR

"OpenLR" package classes

Table A.43— Classes of the "OpenLR" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| OpenlrBaseReferencePoint | Openlr base reference point | Base class used to hold data about a reference point. | D2Class | yes |
| OpenlrLineAttributes | Openlr line attributes | Line attributes are part of a location reference point and consists of functional road class (FRC),form of way (FOW) and bearing (BEAR) data. | D2Class | no |
| OpenlrOffsets | Openlr offsets | Offsets are used to locate the start and end of a location more precisely than bounding to the nodes in a network. | D2Class | no |
| OpenlrPathAttributes | Openlr path attributes | Properties of the path from the associated location reference point to the next location reference point, which are specified to assist correct identification of the point in an external map data source. | D2Class | no |

Associations of "OpenLR" package

Table A.44— Associations of the "OpenLR" package

| Class name | Association end | Designation | Definition | Multiplicity | Target |
| --- | --- | --- | --- | --- | --- |
| OpenlrBaseReferencePoint | openlrLineAttributes | Openlr line attributes | Properties of the line towards the topologically adjacent OpenLR location referencing point, on the shortest path to that point. | 1..1 | OpenlrLineAttributes |
|  | openlrCoordinates | Openlr coordinates | Provides coordinates for the base point of the OpenLR path | 1..1 | PointCoordinates |

"OpenLR" package attributes

Table A.45— Attributes of the "OpenLR" package

| Class name | Attribute name | Designation | Definition | Multiplicity | Type |
| --- | --- | --- | --- | --- | --- |
| OpenlrLineAttributes | openlrBearing | Openlr bearing | defines the bearing field as an integer value between 0 and 359 | 1..1 | AngleInDegrees |
|  | openlrFormOfWay | Openlr form of way | A classification based on the importance of the role that the Road Element (or Ferry Connection) performs in the connectivity of the total road network. (EN ISO 14825 § 7.2.88) | 1..1 | OpenlrFormOfWayEnum |
|  | openlrFunctionalRoadClass | Openlr functional road class | Certain aspects of the physical form that a Road Element takes. It is based on a number of certain physical and traffic properties. (EN ISO 14825 § 7.2.85) | 1..1 | OpenlrFunctionalRoadClassEnum |
| OpenlrOffsets | openlrNegativeOffset | Openlr negative offset | The negative offset along the line of the location measured along the line reference path between the end point of the location reference and the ending node of the line reference path. | 0..1 | MetresAsNonNegativeInteger |
|  | openlrPositiveOffset | Openlr positive offset | The positive offset along the line of the location measured along the line reference path between the start point of the location reference and the starting node of the line reference path. | 0..1 | MetresAsNonNegativeInteger |
| OpenlrPathAttributes | openlrDistanceToNextLRPoint | Openlr distance to next l r point | The DNP attribute measures the distance in meters between two consecutive location reference-points along the location reference path described in the corresponding enumeration | 1..1 | NonNegativeInteger |
|  | openlrLowestFrcToNextLRPoint | Openlr lowest frc to next l r point | The lowest FRC to the next point indicates the lowest functional road class used in the location reference path to the next LR-point. | 1..1 | OpenlrFunctionalRoadClassEnum |

"OpenlrArea" package

D2Payload/LocationReferencing/LocationReference/OpenLR/OpenlrArea

"OpenlrArea" package classes

Table A.46— Classes of the "OpenlrArea" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| OpenlrAreaLocationReference | Openlr area location reference | A two-dimensional part of the surface of the earth which is bounded by a closed curve. An area location may cover parts of the road network but does not necessarily need to. It is represented according to the OpenLR standard for Area Locations | D2Class | yes |
| OpenlrCircleLocationReference | Openlr circle location reference | The OpenLR method of area definition by providing a center position and a radius | D2Class | no |
| OpenlrClosedLineLocationReference | Openlr closed line location reference | The OpenLR method of area definition by providing a closed path (i.e. a circuit) in the road network. The boundary always consists of road segments | D2Class | no |
| OpenlrGridLocationReference | Openlr grid location reference | Area defined using an OpenLR™ method consisting in defining it by a tessellation of rectangles | D2Class | no |
| OpenlrPolygonCorners | Openlr polygon corners | A geodetic coordinate Tuple that defines the vertices of the underlying geometrical polygon. | D2Class | no |
| OpenlrPolygonLocationReference | Openlr polygon location reference | The OpenLR method of area definition by providing points that bound the area | D2Class | no |
| OpenlrRectangle | Openlr rectangle | Area delimited by a rectangle defined by the geodetic co-ordinates of the two ends of its diagonal from south-west to north-east (the rectangle having two sides that are parallel to lines of latitude) | D2Class | no |
| OpenlrRectangleLocationReference | Openlr rectangle location reference | The openLR method of area definition by providing a rectangular shape defined by two geo-coordinate pairs | D2Class | no |

Associations of "OpenlrArea" package

Table A.47— Associations of the "OpenlrArea" package

| Class name | Association end | Designation | Definition | Multiplicity | Target |
| --- | --- | --- | --- | --- | --- |
| OpenlrCircleLocationReference | openlrGeoCoordinate | Openlr geo coordinate |  | 1..1 | OpenlrGeoCoordinate |
| OpenlrClosedLineLocationReference | openlrLocationReferencePoint | Openlr location reference point |  | 1..* | OpenlrLocationReferencePoint |
|  | openlrLastLine | Openlr last line | Provides the line attributes for the last line section closing the polygon. | 1..1 | OpenlrLastLocationReferencePoint |
| OpenlrGridLocationReference | openlrRectangle | Openlr rectangle |  | 1..1 | OpenlrRectangle |
| OpenlrPolygonCorners | openlrCoordinates | Openlr coordinates | Corresponding coordinates of vertices that define a polygon. | 3..* | PointCoordinates |
| OpenlrPolygonLocationReference | openlrPolygonCorners | Openlr polygon corners |  | 1..1 | OpenlrPolygonCorners |
| OpenlrRectangle | openlrLowerLeft | Openlr lower left | The lower left corner of the rectangle. | 1..1 | PointCoordinates |
|  | openlrUpperRight | Openlr upper right | The upper right corner of the rectangle. | 1..1 | PointCoordinates |
| OpenlrRectangleLocationReference | openlrRectangle | Openlr rectangle |  | 1..1 | OpenlrRectangle |

"OpenlrArea" package attributes

Table A.48— Attributes of the "OpenlrArea" package

| Class name | Attribute name | Designation | Definition | Multiplicity | Type |
| --- | --- | --- | --- | --- | --- |
| OpenlrCircleLocationReference | openlrRadius | Openlr radius | The radius of the corresponding circular area. | 1..1 | MetresAsNonNegativeInteger |
| OpenlrGridLocationReference | openlrNumColumns | Openlr num columns | The number that the base rectangle should be multiplied in the east direction | 1..1 | NonNegativeInteger |
|  | openlrNumRows | Openlr num rows | The number that the base rectangle should be multiplied in the north direction | 1..1 | NonNegativeInteger |

"OpenlrLinear" package

D2Payload/LocationReferencing/LocationReference/OpenLR/OpenlrLinear

"OpenlrLinear" package classes

Table A.49— Classes of the "OpenlrLinear" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| OpenlrLinear | Openlr linear | OpenLR line location reference | D2Class | no |
| OpenlrLineLocationReference | Openlr line location reference | A line location reference is defined by an ordered sequence of location reference points and a terminating last location reference point. | D2Class | no |

Associations of "OpenlrLinear" package

Table A.50— Associations of the "OpenlrLinear" package

| Class name | Association end | Designation | Definition | Multiplicity | Target |
| --- | --- | --- | --- | --- | --- |
| OpenlrLineLocationReference | openlrLocationReferencePoint | Openlr location reference point |  | 1..* | OpenlrLocationReferencePoint |
|  | openlrLastLocationReferencePoint | Openlr last location reference point |  | 1..1 | OpenlrLastLocationReferencePoint |
| OpenlrLinear | firstDirection | First direction | First OpenLR reference in first/main direction. | 1..1 | OpenlrLineLocationReference |
|  | oppositeDirection | Opposite direction | If both direction, this is the reference in the opposite direction against firstDirection. | 0..1 | OpenlrLineLocationReference |
|  | openlrOffsets | Openlr offsets | Allows for adding offsets to the line location path defined by nodes when the starting (respectively ending) point does not coincide with a node. | 0..1 | OpenlrOffsets |

"OpenlrLinear" package attributes

There are no defined attributes in the "OpenlrLinear" package.

"OpenlrPoint" package

D2Payload/LocationReferencing/LocationReference/OpenLR/OpenlrPoint

"OpenlrPoint" package classes

Table A.51— Classes of the "OpenlrPoint" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| OpenlrBasePointLocation | Openlr base point location | Holds common data that are used both in OpenlrPointAccessPoint and OpenlrPointAlongLine. | D2Class | yes |
| OpenlrGeoCoordinate | Openlr geo coordinate | A geo-coordinate pair is a position in a map defined by its longitude and latitude coordinate values. | D2Class | no |
| OpenlrLastLocationReferencePoint | Openlr last location reference point | The sequence of location reference points is terminated by a last location reference point. | D2Class | no |
| OpenlrLocationReferencePoint | Openlr location reference point | The basis of a location reference is a sequence of location reference points (LRPs). | D2Class | no |
| OpenlrPointAlongLine | Openlr point along line | Point along a line | D2Class | no |
| OpenlrPointLocationReference | Openlr point location reference | A point location is a zero-dimensional element in a map that specifies a geometric location. | D2Class | yes |
| OpenlrPoiWithAccessPoint | Openlr poi with access point | A point of interest (POI) along a line with access is a point location which is defined by a linear reference path, an offset value (defining the access point) from the starting node of this path and a coordinate pair that defines the POI itself. | D2Class | no |

Associations of "OpenlrPoint" package

Table A.52— Associations of the "OpenlrPoint" package

| Class name | Association end | Designation | Definition | Multiplicity | Target |
| --- | --- | --- | --- | --- | --- |
| OpenlrBasePointLocation | openlrLocationReferencePoint | Openlr location reference point | Allows defining the first point of the OpenLR path | 1..1 | OpenlrLocationReferencePoint |
|  | openlrLastLocationReferencePoint | Openlr last location reference point | Allows defining the last point of the OpenLR path | 1..1 | OpenlrLastLocationReferencePoint |
|  | openlrOffsets | Openlr offsets | Provides optional offsets relative to the path | 0..1 | OpenlrOffsets |
| OpenlrGeoCoordinate | openlrCoordinates | Openlr coordinates | Corresponding coordinates of an OpenLR point defined by its only coordinates. | 1..1 | PointCoordinates |
| OpenlrLocationReferencePoint | openlrPathAttributes | Openlr path attributes | Additional path attributes relative to the next point | 1..1 | OpenlrPathAttributes |
| OpenlrPoiWithAccessPoint | openlrCoordinates | Openlr coordinates | The coordinate of the actual point of interest | 1..1 | PointCoordinates |

"OpenlrPoint" package attributes

Table A.53— Attributes of the "OpenlrPoint" package

| Class name | Attribute name | Designation | Definition | Multiplicity | Type |
| --- | --- | --- | --- | --- | --- |
| OpenlrBasePointLocation | openlrOrientation | Openlr orientation | Orientation of the driving direction in relation with the direction of the underlying linear | 1..1 | OpenlrOrientationEnum |
|  | openlrSideOfRoad | Openlr side of road | Provides the of road where the corresponding point lies. | 1..1 | OpenlrSideOfRoadEnum |

"OperatorAction" package

D2Payload/PayloadPublication/Situation/Classes/OperatorAction

"OperatorAction" package classes

Table A.54— Classes of the "OperatorAction" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| OperatorAction | Operator action | Actions that a traffic operator can decide to implement to prevent or help correct dangerous or poor driving conditions, including maintenance of the road infrastructure. | D2Class | yes |

Associations of "OperatorAction" package

There are no defined associations in the "OperatorAction" package.

"OperatorAction" package attributes

There are no defined attributes in the "OperatorAction" package.

"PointAlongLinearElement" package

D2Payload/LocationReferencing/LocationReference/LinearReferencing/PointAlongLinearElement

"PointAlongLinearElement" package classes

Table A.55— Classes of the "PointAlongLinearElement" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| DistanceAlongLinearElement | Distance along linear element | Distance of a point along a linear element either measured from the start node or a defined referent on that linear element, where the start node is relative to the element definition rather than the direction of traffic flow. | D2Class | yes |
| DistanceFromLinearElementReferent | Distance from linear element referent | Distance of a point along a linear element measured from a "from referent" on the linear element, in the sense relative to the linear element definition rather than the direction of traffic flow or optionally towards a "towards referent". | D2Class | no |
| DistanceFromLinearElementStart | Distance from linear element start | Distance of a point along a linear element measured from the start node of the linear element, where start node is relative to the element definition rather than the direction of traffic flow. | D2Class | no |
| LinearElement | Linear element | A linear element along a single linear object, consistent with EN ISO 19148 definitions. | D2Class | no |
| LinearElementByCode | Linear element by code | A linear element along a single linear object defined by its identifier or code in a road network reference model (specified in LinearElement class) which segments the road network according to specific business rules. | D2Class | no |
| LinearElementByLineString | Linear element by line string | A linear element defined by a line string (class GmlLineString). | D2Class | no |
| LinearElementByPoints | Linear element by points | A linear element along a single linear object defined by its start and end points. | D2Class | no |
| PercentageDistanceAlongLinearElement | Percentage distance along linear element | Distance of a point along a linear element measured from the start node expressed as a percentage of the whole length of the linear element, where start node is relative to the element definition rather than the direction of traffic flow. | D2Class | no |
| PointAlongLinearElement | Point along linear element | A point on a linear element where the linear element is either a part of or the whole of a linear object (i.e. a road), consistent with EN ISO 19148 definitions. | D2Class | no |
| Referent | Referent | A referent on a linear object that has a known location such as a node, a reference marker (e.g. a marker-post), an intersection etc. | D2Class | no |

Associations of "PointAlongLinearElement" package

Table A.56— Associations of the "PointAlongLinearElement" package

| Class name | Association end | Designation | Definition | Multiplicity | Target |
| --- | --- | --- | --- | --- | --- |
| LinearElementByLineString | gmlLineString | Gml line string |  | 1..1 | GmlLineString |
| DistanceFromLinearElementReferent | fromReferent | From referent | A known location along the linear element from which the distanceAlong is measured, termed the "fromReferent" in EN ISO 19148. | 1..1 | Referent |
|  | towardsReferent | Towards referent | A known location along the linear element towards which the distanceAlong is measured, termed the "towardsReferent" in EN ISO 19148. | 0..1 | Referent |
| LinearElementByPoints | endPointOfLinearElement | End point of linear element | The referent at a known location on the linear object which defines the end of the linear element. | 1..1 | Referent |
|  | intermediatePointOnLinearElement | Intermediate point on linear element | A referent at a known location on the linear object which is neither the start or end of the linear element. | 1..1 | Referent |
|  | startPointOfLinearElement | Start point of linear element | The referent at a known location on the linear object which defines the start of the linear element. | 1..1 | Referent |
| PointAlongLinearElement | linearElement | Linear element |  | 1..1 | LinearElement |
|  | distanceAlongLinearElement | Distance along linear element |  | 1..1 | DistanceAlongLinearElement |
| Referent | pointCoordinates | Point coordinates |  | 0..1 | PointCoordinates |

"PointAlongLinearElement" package attributes

Table A.57— Attributes of the "PointAlongLinearElement" package

| Class name | Attribute name | Designation | Definition | Multiplicity | Type |
| --- | --- | --- | --- | --- | --- |
| DistanceFromLinearElementReferent | distanceAlong | Distance along | A measure of distance along a linear element. | 1..1 | MetresAsFloat |
| DistanceFromLinearElementStart | distanceAlong | Distance along | A measure of distance along a linear element. | 1..1 | MetresAsFloat |
| LinearElement | linearElementNature | Linear element nature | An indication of the nature of the linear element. | 0..1 | LinearElementNatureEnum |
|  | linearElementReferenceModel | Linear element reference model | The identifier of a road network reference model which segments the road network according to specific business rules. | 0..1 | String |
|  | linearElementReferenceModelVersion | Linear element reference model version | The version of the identified road network reference model. | 0..1 | String |
|  | roadName | Road name | Name of the road | 0..1 | MultilingualString |
|  | roadNumber | Road number | Identifier/number of the road | 0..1 | String |
| LinearElementByCode | linearElementIdentifier | Linear element identifier | An identifier or code of a linear element (or link) in the road network reference model that is specified in the LinearElement class. | 1..1 | String |
| PercentageDistanceAlongLinearElement | percentageDistanceAlong | Percentage distance along | A measure of distance along a linear element from the start of the element expressed as a percentage of the total length of the linear object. | 1..1 | Percentage |
| PointAlongLinearElement | administrativeAreaOfPoint | Administrative area of point | Identification of the road administration area which contains the specified point. | 0..1 | MultilingualString |
|  | directionAtPoint | Direction at point | The direction of traffic flow at the specified point in terms of general destination direction. | 0..1 | DirectionEnum |
|  | directionRelativeAtPoint | Direction relative at point | The direction of traffic flow at the specified point relative to the direction in which the linear element is defined. | 0..1 | LinearDirectionEnum |
|  | heightGradeOfPoint | Height grade of point | Identification of whether the point on the linear element is at, above or below the normal elevation of a linear element of that type (e.g. road or road section) at that location, typically used to indicate "grade" separation. | 0..1 | HeightGradeEnum |
| Referent | referentDescription | Referent description | Description of the referent. | 0..1 | MultilingualString |
|  | referentIdentifier | Referent identifier | The identifier of the referent, unique on the specified linear element (i.e. road or part of). | 1..1 | String |
|  | referentName | Referent name | The name of the referent, e.g. a junction or intersection name. | 0..1 | String |
|  | referentType | Referent type | The type of the referent. | 1..1 | ReferentTypeEnum |

"PointCoordinates" package

D2Payload/LocationReferencing/LocationReference/PointCoordinates

"PointCoordinates" package classes

Table A.58— Classes of the "PointCoordinates" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| AltitudeConfidence | Altitude confidence | Evaluation of the altitude confidence assessed according to ETSI ISO 102894-2 | D2Class | no |
| HeightCoordinate | Height coordinate | Third coordinate for points defined geodetically | D2Class | no |
| PointByCoordinates | Point by coordinates | A single point defined only by a coordinate set with an optional bearing direction. | D2Class | no |
| PointCoordinates | Point coordinates | A pair of planar coordinates defining the geodetic position of a single point using the European Terrestrial Reference System 1989 (ETRS89). | D2Class | no |
| PositionAccuracy | Position accuracy | Horizontal position accuracy parameters defined according to EN 16803-1 | D2Class | no |
| PositionConfidenceEllipse | Position confidence ellipse | Confidence ellipse position defined in a shape of ellipse with a predefined confidence level (e.g. 95 %). The centre of the ellipse shape corresponds to the reference position point for which the position accuracy is evaluated. | D2Class | no |

Associations of "PointCoordinates" package

Table A.59— Associations of the "PointCoordinates" package

| Class name | Association end | Designation | Definition | Multiplicity | Target |
| --- | --- | --- | --- | --- | --- |
| HeightCoordinate | altitudeConfidence | Altitude confidence |  | 0..1 | AltitudeConfidence |
|  | verticalPositionAccuracy | Vertical position accuracy | Defines the vertical position accuracy according to EN16803-1 | 0..1 | PositionAccuracy |
| PointByCoordinates | pointCoordinates | Point coordinates |  | 1..1 | PointCoordinates |
| PointCoordinates | heightCoordinate | Height coordinate |  | 0..3 | HeightCoordinate |
|  | positionConfidenceEllipse | Position confidence ellipse |  | 0..1 | PositionConfidenceEllipse |
|  | horizontalPositionAccuracy | Horizontal position accuracy | Defines the horizontal position accuracy according EN 16803-1 | 0..1 | PositionAccuracy |

"PointCoordinates" package attributes

Table A.60— Attributes of the "PointCoordinates" package

| Class name | Attribute name | Designation | Definition | Multiplicity | Type |
| --- | --- | --- | --- | --- | --- |
| AltitudeConfidence | altitudeAccuracyCodedError | Altitude accuracy coded error | Error code in case the altitude confidence is out of range or cannot be determined | 0..1 | PositionConfidenceCodedErrorEnum |
|  | altitudeAccuracyCodedValue | Altitude accuracy coded value | Absolute accuracy of reported value of a geographical point for a confidence level expressed by a coded scale. | 0..1 | AltitudeAccuracyEnum |
| HeightCoordinate | heightType | Height type | Type of measured height.When it is omitted it is supposed to be the ellipsoidal height. | 0..1 | HeightTypeEnum |
|  | heightValue | Height value | Value in metres for the height measured vertically at to the planar coordinates the point corresponding. | 1..1 | MetresAsFloat |
| PointByCoordinates | bearing | Bearing | A bearing at the point measured in degrees (0 - 359). Unless otherwise specified the reference direction corresponding to 0 degrees is North. | 0..1 | AngleInDegrees |
| PointCoordinates | latitude | Latitude | Latitude in decimal degrees using the European Terrestrial Reference System 1989 (ETRS89). | 1..1 | Float |
|  | longitude | Longitude | Longitude in decimal degrees using the European Terrestrial Reference System 1989 (ETRS89). | 1..1 | Float |
| PositionAccuracy | accuracyPercentile50 | Accuracy percentile50 | Accuracy defined by the 50th percentile of the cumulative distribution of position errors. | 0..1 | MetresAsFloat |
|  | accuracyPercentile75 | Accuracy percentile75 | Accuracy defined by the 75th percentile of the cumulative distribution of position errors | 0..1 | MetresAsFloat |
|  | accuracyPercentile95 | Accuracy percentile95 | Accuracy defined by the 95th percentile of the cumulative distribution of position errors | 0..1 | MetresAsFloat |
| PositionConfidenceEllipse | semiMajorAxisLength | Semi major axis length | Half of length of the major axis, i.e. distance between the centre point and major axis point of the position accuracy ellipse. | 0..1 | MetresAsFloat |
|  | semiMajorAxisLengthCodedError | Semi major axis length coded error | Provides a coded error in case the semi-major axis length is not defined | 0..1 | PositionConfidenceCodedErrorEnum |
|  | semiMajorAxisOrientation | Semi major axis orientation | Orientation direction of the ellipse major axis of the position accuracy ellipse with regards to the geographic north. | 0..1 | AngleInDegrees |
|  | semiMajorAxisOrientationError | Semi major axis orientation error | Indicates whether the ellipse orientation is unavailable (True) or not (False) | 0..1 | Boolean |
|  | semiMinorAxisLength | Semi minor axis length | Half of length of the minor axis, i.e. distance between the centre point and minor axis point of the position accuracy ellipse | 0..1 | MetresAsFloat |
|  | semiMinorAxisLengthCodedError | Semi minor axis length coded error | Provides a coded error in case the semi-minor axis length is not defined | 0..1 | PositionConfidenceCodedErrorEnum |

"Roadworks" package

D2Payload/PayloadPublication/Situation/Classes/OperatorAction/Roadworks

"Roadworks" package classes

Table A.61— Classes of the "Roadworks" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| ConstructionWorks | Construction works | Roadworks involving the construction of new infrastructure. | D2Class | no |
| MaintenanceVehicles | Maintenance vehicles | Details of the maintenance vehicles involved in the roadworks activity. | D2Class | no |
| MaintenanceWorks | Maintenance works | Roadworks involving the maintenance or installation of infrastructure. | D2Class | no |
| Roadworks | Roadworks | Highway maintenance, installation and construction activities that may potentially affect traffic operations. | D2Class | yes |

Associations of "Roadworks" package

Table A.62— Associations of the "Roadworks" package

| Class name | Association end | Designation | Definition | Multiplicity | Target |
| --- | --- | --- | --- | --- | --- |
| Roadworks | maintenanceVehicles | Maintenance vehicles |  | 0..1 | MaintenanceVehicles |

"Roadworks" package attributes

Table A.63— Attributes of the "Roadworks" package

| Class name | Attribute name | Designation | Definition | Multiplicity | Type |
| --- | --- | --- | --- | --- | --- |
| ConstructionWorks | constructionWorkType | Construction work type | The type of construction work being performed. | 1..1 | ConstructionWorkTypeEnum |
| MaintenanceVehicles | maintenanceVehicleActions | Maintenance vehicle actions | The actions of the maintenance vehicles associated with the roadworks activities. | 0..* | MaintenanceVehicleActionsEnum |
| MaintenanceWorks | roadMaintenanceType | Road maintenance type | The type of road maintenance or installation work at the specified location. | 1..* | RoadMaintenanceTypeEnum |

"SituationPublication" package

D2Payload/PayloadPublication/Situation/SituationPublication

"SituationPublication" package classes

Table A.64— Classes of the "SituationPublication" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| SituationPublication | Situation publication | A publication containing zero or more traffic/travel situations. | D2Class | no |

Associations of "SituationPublication" package

Table A.65— Associations of the "SituationPublication" package

| Class name | Association end | Designation | Definition | Multiplicity | Target |
| --- | --- | --- | --- | --- | --- |
| SituationPublication | situation | Situation |  | 0..* | Situation |

"SituationPublication" package attributes

There are no defined attributes in the "SituationPublication" package.

"SupplementaryPositionalDescription" package

D2Payload/LocationReferencing/LocationReference/SupplementaryPositionalDescription

"SupplementaryPositionalDescription" package classes

Table A.66— Classes of the "SupplementaryPositionalDescription" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| Carriageway | Carriageway | Supplementary positional information which details carriageway and lane locations. Several instances may exist where the element being described extends over more than one carriageway. | D2Class | no |
| Lane | Lane | Indicates a specific lane or group of lanes. | D2Class | no |
| RoadInformation | Road information | Information on a road | D2Class | no |
| SupplementaryPositionalDescription | Supplementary positional description | A collection of supplementary positional information which improves the precision of the location. | D2Class | no |

Associations of "SupplementaryPositionalDescription" package

Table A.67— Associations of the "SupplementaryPositionalDescription" package

| Class name | Association end | Designation | Definition | Multiplicity | Target |
| --- | --- | --- | --- | --- | --- |
| Carriageway | lane | Lane |  | 0..* | Lane |
| SupplementaryPositionalDescription | carriageway | Carriageway |  | 0..* | Carriageway |
|  | namedArea | Named area |  | 0..1 | NamedArea |
|  | roadInformation | Road information | Information on a set of one or more roads. The location could correspond to a part of the road identified, the whole stretch of road identified, or a combination of multiple road sections. | 0..* | RoadInformation |

"SupplementaryPositionalDescription" package attributes

Table A.68— Attributes of the "SupplementaryPositionalDescription" package

| Class name | Attribute name | Designation | Definition | Multiplicity | Type |
| --- | --- | --- | --- | --- | --- |
| Carriageway | carriageway | Carriageway | Indicates the section of carriageway to which the location relates. | 1..1 | CarriagewayEnum |
|  | originalNumberOfLanes | Original number of lanes | Normal number of lanes, potentially available for moving traffic, before reduction due to situations. Hard shoulder should not be counted unless it is sometimes used operationally for moving traffic. | 0..1 | Integer |
| Lane | laneNumber | Lane number | The number of the lane, where 1 is nearest the hard shoulder/verge and the numbers increase towards the central reservation/road axis. | 0..1 | Integer |
|  | laneUsage | Lane usage | Indicates the specific lane to which the location relates. | 0..1 | LaneEnum |
| RoadInformation | roadDestination | Road destination | A destination associated with this road. | 0..1 | String |
|  | roadName | Road name | The name of the road | 0..1 | String |
|  | roadNumber | Road number | The road number designated by the road authority | 0..1 | String |
| SupplementaryPositionalDescription | directionPurpose | Direction purpose | Identifies the main purpose of the road at the location | 0..1 | DirectionPurposeEnum |
|  | geographicDescriptor | Geographic descriptor | Descriptor which identifies a geographic characteristic to help identify the specific location | 0..1 | GeographicCharacteristicEnum |
|  | infrastructureDescriptor | Infrastructure descriptor | Descriptor which identifies infrastructure to help identify the specific location. | 0..1 | InfrastructureDescriptorEnum |
|  | lengthAffected | Length affected | This indicates the length (measured in metres) of carriageway (and lanes) affected by the associated traffic element. | 0..1 | MetresAsFloat |
|  | locationDescription | Location description | Supplementary human-readable description of the location | 0..1 | MultilingualString |
|  | locationPrecision | Location precision | Indicates that the location is given with a precision which is better than the stated value in metres. | 0..1 | MetresAsNonNegativeInteger |
|  | positionOnCarriageway | Position on carriageway | Relative position across carriageway | 0..1 | RelativePositionOnCarriagewayEnum |
|  | sequentialRampNumber | Sequential ramp number | The sequential number of an exit/entrance ramp from a given location in a given direction (normally used to indicate a specific exit/entrance in a complex junction/intersection). | 0..1 | NonNegativeInteger |

"TpegAreaLocation" package

D2Payload/LocationReferencing/LocationReference/TpegLoc/TpegAreaLocation

"TpegAreaLocation" package classes

Table A.69— Classes of the "TpegAreaLocation" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| TpegAreaLocation | TPEG area location | A geographic or geometric area defined by a TPEG-Loc structure which may include height information for additional geospatial discrimination. | D2Class | yes |
| TpegGeometricArea | TPEG geometric area | A geometric area defined by a centre point and a radius. | D2Class | no |
| TpegHeight | TPEG height | Height information which provides additional discrimination for the applicable area. | D2Class | no |
| TpegNamedOnlyArea | TPEG named only area | An area defined by a well-known name. | D2Class | no |

Associations of "TpegAreaLocation" package

Table A.70— Associations of the "TpegAreaLocation" package

| Class name | Association end | Designation | Definition | Multiplicity | Target |
| --- | --- | --- | --- | --- | --- |
| TpegAreaLocation | tpegHeight | TPEG height |  | 0..1 | TpegHeight |
| TpegGeometricArea | centrePoint | Centre point | Centre point of a circular geometric area. | 1..1 | PointCoordinates |
|  | name | Name | Name of area. | 0..1 | TpegAreaDescriptor |
| TpegNamedOnlyArea | name | Name | Name of area. | 1..* | TpegAreaDescriptor |

"TpegAreaLocation" package attributes

Table A.71— Attributes of the "TpegAreaLocation" package

| Class name | Attribute name | Designation | Definition | Multiplicity | Type |
| --- | --- | --- | --- | --- | --- |
| TpegAreaLocation | tpegAreaLocationType | TPEG area location type | The type of TPEG location. | 1..1 | TpegLoc01AreaLocationSubtypeEnum |
| TpegGeometricArea | radius | Radius | The radius of the corresponding circular area | 1..1 | MetresAsNonNegativeInteger |
| TpegHeight | height | Height | A measurement of height in metres | 0..1 | MetresAsFloat |
|  | heightType | Height type | A descriptive identification of relative height using TPEG-Loc location referencing. | 1..1 | TpegLoc04HeightTypeEnum |

"TpegDescriptor" package

D2Payload/LocationReferencing/LocationReference/TpegLoc/TpegDescriptor

"TpegDescriptor" package classes

Table A.72— Classes of the "TpegDescriptor" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| TpegAreaDescriptor | TPEG area descriptor | A descriptor for describing an area location. | D2Class | no |
| TpegDescriptor | TPEG descriptor | A collection of information providing descriptive references to locations using the TPEG-Loc location referencing approach. | D2Class | yes |
| TpegIlcPointDescriptor | TPEG ILC point descriptor | A descriptor for describing a junction by defining the intersecting roads. | D2Class | no |
| TpegJunctionPointDescriptor | TPEG junction point descriptor | A descriptor for describing a point at a junction on a road network. | D2Class | no |
| TpegOtherPointDescriptor | TPEG other point descriptor | General descriptor for describing a point. | D2Class | no |
| TpegPointDescriptor | TPEG point descriptor | A descriptor for describing a point location. | D2Class | yes |

Associations of "TpegDescriptor" package

There are no defined associations in the "TpegDescriptor" package.

"TpegDescriptor" package attributes

Table A.73— Attributes of the "TpegDescriptor" package

| Class name | Attribute name | Designation | Definition | Multiplicity | Type |
| --- | --- | --- | --- | --- | --- |
| TpegAreaDescriptor | tpegAreaDescriptorType | TPEG area descriptor type | The nature of the descriptor used to define the location under consideration (derived from the TPEG Loc table 03). | 1..1 | TpegLoc03AreaDescriptorSubtypeEnum |
| TpegDescriptor | descriptor | Descriptor | A text string which describes or elaborates the location. | 1..1 | MultilingualString |
| TpegIlcPointDescriptor | tpegIlcPointDescriptorType | TPEG ILC point descriptor type | The nature of the descriptor used to define the location under consideration (derived from the TPEG Loc table 03). | 1..1 | TpegLoc03IlcPointDescriptorSubtypeEnum |
| TpegJunctionPointDescriptor | tpegJunctionPointDescriptorType | TPEG junction point descriptor type | The nature of the descriptor used to define the location under consideration (derived from the TPEG Loc table 03). | 1..1 | TpegLoc03JunctionPointDescriptorSubtypeEnum |
| TpegOtherPointDescriptor | tpegOtherPointDescriptorType | TPEG other point descriptor type | The nature of the descriptor used to define the location under consideration (derived from the TPEG Loc table 03). | 1..1 | TpegLoc03OtherPointDescriptorSubtypeEnum |

"TpegLinearLocation" package

D2Payload/LocationReferencing/LocationReference/TpegLoc/TpegLinearLocation

"TpegLinearLocation" package classes

Table A.74— Classes of the "TpegLinearLocation" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| TpegLinearLocation | TPEG linear location | A linear section along a single road defined between two points on the same road by a TPEG-Loc structure. | D2Class | no |

Associations of "TpegLinearLocation" package

Table A.75— Associations of the "TpegLinearLocation" package

| Class name | Association end | Designation | Definition | Multiplicity | Target |
| --- | --- | --- | --- | --- | --- |
| TpegLinearLocation | from | From | The location at the up stream end of the linear section of road. | 1..1 | TpegPoint |
|  | to | To | The location at the down stream end of the linear section of road. | 1..1 | TpegPoint |

"TpegLinearLocation" package attributes

Table A.76— Attributes of the "TpegLinearLocation" package

| Class name | Attribute name | Designation | Definition | Multiplicity | Type |
| --- | --- | --- | --- | --- | --- |
| TpegLinearLocation | tpegDirection | TPEG direction | The direction of traffic flow. | 1..1 | DirectionEnum |
|  | tpegLinearLocationType | TPEG linear location type | The type of TPEG location. | 1..1 | TpegLoc01LinearLocationSubtypeEnum |

"TpegPointLocation" package

D2Payload/LocationReferencing/LocationReference/TpegLoc/TpegPointLocation

"TpegPointLocation" package classes

Table A.77— Classes of the "TpegPointLocation" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| TpegFramedPoint | TPEG framed point | A point on the road network which is framed between two other points on the same road. | D2Class | no |
| TpegJunction | TPEG junction | A point on the road network which is a road junction point. | D2Class | no |
| TpegNonJunctionPoint | TPEG non junction point | A point on the road network which is not a road junction point. | D2Class | no |
| TpegPoint | TPEG point | A point on the road network which is either a junction point or a non junction point. | D2Class | yes |
| TpegPointLocation | TPEG point location | A single point on the road network defined by a TPEG-Loc structure and which has an associated direction of traffic flow. | D2Class | yes |
| TpegSimplePoint | TPEG simple point | A point on the road network which is not bounded by any other points on the road network. | D2Class | no |

Associations of "TpegPointLocation" package

Table A.78— Associations of the "TpegPointLocation" package

| Class name | Association end | Designation | Definition | Multiplicity | Target |
| --- | --- | --- | --- | --- | --- |
| TpegFramedPoint | framedPoint | Framed point | A single non-junction point on the road network which is framed between two other specified points on the road network. | 1..1 | TpegNonJunctionPoint |
|  | to | To | The location at the downstream end of the section of road which frames the TPEGFramedPoint. | 1..1 | TpegPoint |
|  | from | From | The location at the upstream end of the section of road which frames the TPEGFramedPoint. | 1..1 | TpegPoint |
| TpegSimplePoint | point | Point | A single point defined by a coordinate set and TPEG descriptors. | 1..1 | TpegPoint |
| TpegJunction | pointCoordinates | Point coordinates |  | 1..1 | PointCoordinates |
|  | name | Name | A name which identifies a junction point on the road network | 0..1 | TpegJunctionPointDescriptor |
|  | ilc | Ilc | A descriptor for describing a junction by identifying the intersecting roads at a road junction. | 1..3 | TpegIlcPointDescriptor |
|  | otherName | Other name | A descriptive name which helps to identify the junction point. | 0..* | TpegOtherPointDescriptor |
| TpegNonJunctionPoint | pointCoordinates | Point coordinates |  | 1..1 | PointCoordinates |
|  | name | Name | A descriptive name which helps to identify the non-junction point. At least one descriptor must identify the road on which the point is located, i.e. must be of type 'linkName' or 'localLinkName'. | 1..* | TpegOtherPointDescriptor |

"TpegPointLocation" package attributes

Table A.79— Attributes of the "TpegPointLocation" package

| Class name | Attribute name | Designation | Definition | Multiplicity | Type |
| --- | --- | --- | --- | --- | --- |
| TpegFramedPoint | tpegFramedPointLocationType | TPEG framed point location type | The type of TPEG location. | 1..1 | TpegLoc01FramedPointLocationSubtypeEnum |
| TpegPointLocation | tpegDirection | TPEG direction | The direction of traffic flow. | 1..1 | DirectionEnum |
| TpegSimplePoint | tpegSimplePointLocationType | TPEG simple point location type | The type of TPEG location. | 1..1 | TpegLoc01SimplePointLocationSubtypeEnum |

"TrafficElement" package

D2Payload/PayloadPublication/Situation/Classes/TrafficElement

"TrafficElement" package classes

Table A.80— Classes of the "TrafficElement" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| TrafficElement | Traffic element | An event which is not planned by the traffic operator, which is affecting, or has the potential to affect traffic flow. | D2Class | yes |

Associations of "TrafficElement" package

There are no defined associations in the "TrafficElement" package.

"TrafficElement" package attributes

Table A.81— Attributes of the "TrafficElement" package

| Class name | Attribute name | Designation | Definition | Multiplicity | Type |
| --- | --- | --- | --- | --- | --- |
| TrafficElement | trafficConstrictionType | Traffic constriction type | The type of constriction to which traffic is subjected as a result of an unplanned event | 0..1 | TrafficConstrictionTypeEnum |

"Validity" package

D2Payload/Common/Classes/Validity

"Validity" package classes

Table A.82— Classes of the "Validity" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| NamedArea | Named area | An abstract hook class to hook in a model for a named area. | D2Class | yes |
| OverallPeriod | Overall period | A continuous or discontinuous period of validity defined by overall bounding start and end times and the possible intersection of valid periods (potentially recurring) with the complement of exception periods (also potentially recurring). | D2Class | no |
| Validity | Validity | Specification of validity, either explicitly or by a validity time period specification which may be discontinuous. | D2Class | no |

Associations of "Validity" package

Table A.83— Associations of the "Validity" package

| Class name | Association end | Designation | Definition | Multiplicity | Target |
| --- | --- | --- | --- | --- | --- |
| Validity | validityTimeSpecification | Validity time specification | A specification of periods of validity defined by overall bounding start and end times and the possible intersection of valid periods with exception periods (exception periods overriding valid periods). | 1..1 | OverallPeriod |

"Validity" package attributes

Table A.84— Attributes of the "Validity" package

| Class name | Attribute name | Designation | Definition | Multiplicity | Type |
| --- | --- | --- | --- | --- | --- |
| OverallPeriod | overallStartTime | Overall start time | Start of bounding period of validity defined by date and time. | 1..1 | DateTime |
| Validity | validityStatus | Validity status | Specification of validity, either explicitly overriding the validity time specification or confirming it. | 1..1 | ValidityStatusEnum |

"Vehicle" package

D2Payload/Common/Classes/Vehicle

"Vehicle" package classes

Table A.85— Classes of the "Vehicle" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| Vehicle | Vehicle | Details of an individual vehicle. | D2Class | no |

Associations of "Vehicle" package

Table A.86— Associations of the "Vehicle" package

| Class name | Association end | Designation | Definition | Multiplicity | Target |
| --- | --- | --- | --- | --- | --- |
| Vehicle | vehicleCharacteristics | Vehicle characteristics |  | 0..1 | VehicleCharacteristics |

"Vehicle" package attributes

Table A.87— Attributes of the "Vehicle" package

| Class name | Attribute name | Designation | Definition | Multiplicity | Type |
| --- | --- | --- | --- | --- | --- |
| Vehicle | vehicleStatus | Vehicle status | Vehicle status. | 0..1 | VehicleStatusEnum |

"VehicleCharacteristics" package

D2Payload/Common/Classes/VehicleCharacteristics

"VehicleCharacteristics" package classes

Table A.88— Classes of the "VehicleCharacteristics" package

| Class name | Designation | Definition | Stereotype | Abstract |
| --- | --- | --- | --- | --- |
| VehicleCharacteristics | Vehicle characteristics | The characteristics of a vehicle, e.g. lorry of gross weight greater than 30 tonnes. | D2Class | no |

Associations of "VehicleCharacteristics" package

There are no defined associations in the "VehicleCharacteristics" package.

"VehicleCharacteristics" package attributes

Table A.89— Attributes of the "VehicleCharacteristics" package

| Class name | Attribute name | Designation | Definition | Multiplicity | Type |
| --- | --- | --- | --- | --- | --- |
| VehicleCharacteristics | vehicleType | Vehicle type | Vehicle type. | 0..* | VehicleTypeEnum |

Data Dictionary of <<D2Datatype>> for ""

This clause contains the definitions of all data types which are used in the "".

The <<D2Datatype>> "AlertCLocationCode"

A positive integer number (between 1 and 63 487) which uniquely identifies a pre-defined Alert C location defined within an Alert-C table.

The <<D2Datatype>> "AngleInDegrees"

An integer number representing an angle in whole degrees between 0 and 359.

The <<D2Datatype>> "Boolean"

Boolean has the value space required to support the mathematical concept of binary-valued logic: {true, false}.

The <<D2Datatype>> "CountryCode"

EN ISO 3166-1 alpha-2 two-letter country code

The <<D2Datatype>> "DateTime"

A combination of integer-valued year, month, day, hour, minute properties, a decimal-valued second property and a time zone property from which it is possible to determine the local time, the equivalent UTC time and the time zone offset from UTC.

The <<D2Datatype>> "Float"

A floating point number whose value space consists of the values m × 2^e, where m is an integer whose absolute value is less than 2^24, and e is an integer between -149 and 104, inclusive.

The <<D2Datatype>> "GmlPosList"

List of coordinates, space-separated, within the same coordinate reference system, defining a geometric entity. Modelled on DirectPositionListType in GML (EN ISO 19136), but constrained to represent a 2D or 3D polyline.

The <<D2Datatype>> "Integer"

An integer number whose value space is the set {-2147483648, -2147483647, -2147483646, ..., -2, -1, 0, 1, 2, ..., 2147483645, 2147483646, 2147483647}.

The <<D2Datatype>> "KilometresPerHour"

A measure of speed defined in kilometres per hour.

The <<D2Datatype>> "Language"

A language datatype, identifies a specified language by an ISO 639-1 2-alpha code.

The <<D2Datatype>> "LongString"

A character string with no specified length limit, whose value space is the set of finite-length sequences of characters. Every character has a corresponding Universal Character Set code point (as defined in ISO/IEC 10646), which is an integer.

The <<D2Datatype>> "MetresAsFloat"

A measure of distance defined in metres in a floating point format.

The <<D2Datatype>> "MetresAsNonNegativeInteger"

A measure of distance defined in metres in a non negative integer format.

The <<D2Datatype>> "MultilingualString"

A multilingual string, whereby the same text may be expressed in more than one language.

The <<D2Datatype>> "NonNegativeInteger"

An integer number whose value space is the set {0, 1, 2, ..., 2147483645, 2147483646, 2147483647}.

The <<D2Datatype>> "NutsCode"

A NUTS code (Nomenclature of territorial units for statistics).

The <<D2Datatype>> "Percentage"

A measure of percentage.

The <<D2Datatype>> "String"

A character string whose value space is the set of finite-length sequences of characters. Every character has a corresponding Universal Character Set code point (as defined in ISO/IEC 10646), which is an integer.

The <<D2Datatype>> "SubdivisionCode"

ISO 3166-2 country sub-division code (up to 3 characters).

The <<D2Datatype>> "VehiclesPerHour"

Vehicles per hour.

Data Dictionary of <<D2Enumeration>> for ""

This clause contains the definitions of all enumerations which are used in the "".

The <<D2Enumeration>> "AccidentTypeEnum"

Collection of descriptive terms for types of accidents.

Table A.90— Values contained in the enumeration "AccidentTypeEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| accident | Accident | Accidents are situations in which one or more vehicles lose control and do not recover. They include collisions between vehicle(s) or other road user(s), between vehicle(s) and fixed obstacle(s), or they result from a vehicle running off the road. |
| accidentInvolvingHazardousMaterials | Accident involving hazardous materials | Includes all accidents involving at least one vehicle believed to be carrying materials, which could present an additional hazard to road users. |
| accidentInvolvingHeavyLorries | Accident involving heavy lorries | Includes all accidents involving at least one heavy goods vehicle. |
| collision | Collision | Collision of vehicle with another object of unspecified type. |
| multipleVehicleAccident | Multiple vehicle accident | Includes all accidents involving three or more vehicles. |

The <<D2Enumeration>> "AlertCDirectionEnum"

Direction used to reach the primary location from the secondary location in ALERT-C location table, as defined in CEN ISO 14819-1

Table A.91— Values contained in the enumeration "AlertCDirectionEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| negative | Negative | The direction of navigation in an ALERT-C table that corresponds to the negative offset usage to go from the secondary location to the primary location within the ALERT-C location table. |
| positive | Positive | The direction of navigation in an ALERT-C table that corresponds to the negative offset usage to go from the secondary location to the primary location within the ALERT-C location table. |

The <<D2Enumeration>> "AltitudeAccuracyEnum"

Coded level of vertical accuracy

Table A.92— Values contained in the enumeration "AltitudeAccuracyEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| equalToOrLessThan100Metres | Equal to or less than100 metres | Indicates if the altitude accuracy is equal to or less than 100 metres |
| equalToOrLessThan10Centimetres | Equal to or less than10 centimetres | Indicates if the altitude accuracy is equal to or less than 10 centimetres |
| equalToOrLessThan10Metres | Equal to or less than10 metres | Indicates if the altitude accuracy is equal to or less than 10 metres |
| equalToOrLessThan1Centimetre | Equal to or less than1 centimetre | Indicates if the altitude accuracy is equal to or less than 1 centimetre |
| equalToOrLessThan1Metre | Equal to or less than1 metre | Indicates if the altitude accuracy is equal to or less than 1 metre |
| equalToOrLessThan200Metres | Equal to or less than200 metres | Indicates if the altitude accuracy is equal to or less than 200 metres |
| equalToOrLessThan20Centimetres | Equal to or less than20 centimetres | Indicates if the altitude accuracy is equal to or less than 20 centimetres |
| equalToOrLessThan20Metres | Equal to or less than20 metres | Indicates if the altitude accuracy is equal to or less than 20 metres |
| equalToOrLessThan2Centimetres | Equal to or less than2 centimetres | Indicates if the altitude accuracy is equal to or less than 2 centimetres |
| equalToOrLessThan2Metres | Equal to or less than2 metres | Indicates if the altitude accuracy is equal to or less than 2 metres |
| equalToOrLessThan50Centimetres | Equal to or less than50 centimetres | Indicates if the altitude accuracy is equal to or less than 50 centimetres |
| equalToOrLessThan50Metres | Equal to or less than50 metres | Indicates if the altitude accuracy is equal to or less than 50 metres |
| equalToOrLessThan5Centimetres | Equal to or less than5 centimetres | Indicates if the altitude accuracy is equal to or less than 5 centimetres |
| equalToOrLessThan5Metres | Equal to or less than5 metres | Indicates if the altitude accuracy is equal to or less than 5 metres |

The <<D2Enumeration>> "AnimalPresenceTypeEnum"

Types of animal presence.

Table A.93— Values contained in the enumeration "AnimalPresenceTypeEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| animalsOnTheRoad | Animals on the road | Traffic may be disrupted due to animals on the roadway. |
| herdOfAnimalsOnTheRoad | Herd of animals on the road | Traffic may be disrupted due to a herd of animals on the roadway. |
| largeAnimalsOnTheRoad | Large animals on the road | Traffic may be disrupted due to large animals on the roadway. |

The <<D2Enumeration>> "AreaPlacesEnum"

Type of area place(s)

Table A.94— Values contained in the enumeration "AreaPlacesEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| atBorders | At borders | At national borders |
| atHighAltitudes | At high altitudes | At high altitudes |
| inBuiltUpAreas | In built up areas | In built up areas, i.e. villages, towns and cities |
| inForestedAreas | In forested areas | On sections of the road where it runs through or adjacent to forested areas |
| inGalleries | In galleries | In galleries |
| inLowLyingAreas | In low lying areas | In low-lying areas |
| inRuralAreas | In rural areas | In rural areas, i.e. outside villages, towns and cities |
| inShadedAreas | In shaded areas | In shaded areas |
| inTheInnerCityAreas | In the inner city areas | In the city centre areas |
| inTunnels | In tunnels | In tunnels |
| onBridges | On bridges | On bridges |
| onDownhillSections | On downhill sections | On downhill sections of the road |
| onElevatedSections | On elevated sections | On elevated sections of the road |
| onEnteringOrLeavingTunnels | On entering or leaving tunnels | On entering or leaving tunnels |
| onFlyovers | On flyovers | On flyover sections of the road, i.e. sections of the road which pass over another road |
| onPasses | On passes | On mountain passes |
| onUndergroundSections | On underground sections | On underground sections of the road |
| onUnderpasses | On underpasses | On underpasses, i.e. sections of the road which pass under another road |

The <<D2Enumeration>> "CarriagewayEnum"

List of descriptors identifying specific carriageway details.

Table A.95— Values contained in the enumeration "CarriagewayEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| connectingCarriageway | Connecting carriageway | On the connecting carriageway. |
| cycleTrack | Cycle track | Independent road or part of a road designated for cycles, signposted as such. A cycle track is separated from other roads or other parts of the same road by structural means. |
| entrySlipRoad | Entry slip road | On the entry slip road. |
| exitSlipRoad | Exit slip road | On the exit slip road. |
| flyover | Flyover | On the flyover, i.e. the section of road passing over another. |
| footpath | Footpath | On the footpath |
| leftHandFeederRoad | Left hand feeder road | On the left hand feeder road. |
| leftHandParallelCarriageway | Left hand parallel carriageway | On the left hand parallel carriageway. |
| mainCarriageway | Main carriageway | On the main carriageway. |
| oppositeCarriageway | Opposite carriageway | On the opposite carriageway. |
| parallelCarriageway | Parallel carriageway | On the adjacent external parallel carriageway. |
| rightHandFeederRoad | Right hand feeder road | On the right hand feeder road. |
| rightHandParallelCarriageway | Right hand parallel carriageway | On the right hand parallel carriageway. |
| roundabout | Roundabout | On the roundabout. |
| serviceRoad | Service road | On the adjacent service road. |
| slipRoads | Slip roads | On the slip roads. |
| underpass | Underpass | On the underpass, i.e. the section of road passing under another. |
| unspecifiedCarriageway | Unspecified carriageway | On an unspecified carriageway |

The <<D2Enumeration>> "ComputationMethodEnum"

Types of computational methods used in deriving data values for data sets.

Table A.96— Values contained in the enumeration "ComputationMethodEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| arithmeticAverageOfSamplesBasedOnAFixedNumberOfSamples | Arithmetic average of samples based on a fixed number of samples | Arithmetic average of sample values based on a fixed number of samples. |
| arithmeticAverageOfSamplesInATimePeriod | Arithmetic average of samples in a time period | Arithmetic average of sample values in a time period. |
| harmonicAverageOfSamplesInATimePeriod | Harmonic average of samples in a time period | Harmonic average of sample values in a time period. |
| medianOfSamplesInATimePeriod | Median of samples in a time period | Median of sample values taken over a time period. |
| movingAverageOfSamples | Moving average of samples | Moving average of sample values. |

The <<D2Enumeration>> "ConstructionWorkTypeEnum"

Types of works relating to construction.

Table A.97— Values contained in the enumeration "ConstructionWorkTypeEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| blastingWork | Blasting work | Blasting or quarrying work at the specified location. |

The <<D2Enumeration>> "DirectionEnum"

List of directions of travel.

Table A.98— Values contained in the enumeration "DirectionEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| aligned | Aligned | Same direction as the normal direction of flow at this point on the road network. |
| allDirections | All directions | All directions (where more than two are applicable) at this point on the road network. |
| anticlockwise | Anticlockwise | Anti-clockwise. |
| bothWays | Both ways | Both directions that are applicable at this point on the road network. |
| clockwise | Clockwise | Clockwise. |
| eastBound | East bound | East bound general direction. |
| inboundTowardsTown | Inbound towards town | Heading towards town centre direction of travel. |
| innerRing | Inner ring | Inner ring direction. |
| northBound | North bound | North bound general direction. |
| northEastBound | North east bound | North east bound general direction. |
| northWestBound | North west bound | North west bound general direction. |
| opposite | Opposite | Opposite direction to the normal direction of flow at this point on the road network. |
| other | Other | Other than as defined in this enumeration. |
| outboundFromTown | Outbound from town | Heading out of or away from the town centre direction of travel. |
| outerRing | Outer ring | Outer ring direction. |
| southBound | South bound | South bound general direction. |
| southEastBound | South east bound | South east bound general direction. |
| southWestBound | South west bound | South west bound general direction. |
| unknown | Unknown | Direction is unknown. |
| westBound | West bound | West bound general direction. |

The <<D2Enumeration>> "DirectionPurposeEnum"

Main purpose of a direction of a road

Table A.99— Values contained in the enumeration "DirectionPurposeEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| inbound | Inbound | On the carriageway or lane which is inbound towards the centre of the town or city. |
| outbound | Outbound | On the carriageway or lane which is outbound from the centre of the town or city. |

The <<D2Enumeration>> "EnvironmentalObstructionTypeEnum"

Types of environmental obstructions.

Table A.100— Values contained in the enumeration "EnvironmentalObstructionTypeEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| avalanches | Avalanches | The road may be obstructed or partially obstructed due to snow slides. |
| fallenTrees | Fallen trees | The road is obstructed or partially obstructed by one or more fallen trees. |
| landslips | Landslips | The road may be obstructed or partially obstructed due to landslides. |
| rockfalls | Rockfalls | The road may be obstructed or partially obstructed due to fallen rocks. |

The <<D2Enumeration>> "GeographicCharacteristicEnum"

Descriptor to help to identify a specific location.

Table A.101— Values contained in the enumeration "GeographicCharacteristicEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| aroundABendInRoad | Around a bend in road | Around a bend in the road. |
| onBorder | On border | On border crossing. |
| onPass | On pass | On mountain pass. |
| overCrestOfHill | Over crest of hill | Over the crest of a hill. |

The <<D2Enumeration>> "HeightGradeEnum"

List of height or vertical gradings of road sections.

Table A.102— Values contained in the enumeration "HeightGradeEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| aboveGrade | Above grade | Above or over the normal road grade elevation. |
| atGrade | At grade | At the normal road grade elevation. |
| belowGrade | Below grade | Below or under the normal road grade elevation. |

The <<D2Enumeration>> "HeightTypeEnum"

Coded value for type of height

Table A.103— Values contained in the enumeration "HeightTypeEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| ellipsoidalHeight | Ellipsoidal height | Value measured vertically above the reference ellipsoid |
| gravityRelatedHeight | Gravity related height | Height type corresponding a value measured along direction of gravity above the reference geoid i.e. equipotential surface of the Earth's gravity field which globally approximates mean sea level. |
| relativeHeight | Relative height | Height type corresponding to value masured vertically above the ground level at this point. |

The <<D2Enumeration>> "InformationStatusEnum"

Status of the related information (i.e. real, test or exercise).

Table A.104— Values contained in the enumeration "InformationStatusEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| real | Real | The information is real. It is not a test or exercise. |
| securityExercise | Security exercise | The information is part of an exercise which is for testing security. |
| technicalExercise | Technical exercise | The information is part of an exercise which includes tests of associated technical subsystems. |
| test | Test | The information is part of a test for checking the exchange of this type of information. |

The <<D2Enumeration>> "InfrastructureDescriptorEnum"

Descriptor identifying infrastructure to help to identify a specific location.

Table A.105— Values contained in the enumeration "InfrastructureDescriptorEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| atMotorwayInterchange | At motorway interchange | At a motorway interchange. |
| atRestArea | At rest area | At rest area off the carriageway. |
| atServiceArea | At service area | At service area. |
| atTollPlaza | At toll plaza | At toll plaza. |
| atTunnelEntryOrExit | At tunnel entry or exit | At entry or exit of tunnel. |
| inGallery | In gallery | In gallery. |
| inTunnel | In tunnel | In tunnel. |
| onBridge | On bridge | On bridge. |
| onConnector | On connector | On connecting carriageway between two different roads or road sections. |
| onElevatedSection | On elevated section | On elevated section of road. |
| onFlyover | On flyover | On flyover, i.e. on section of road over another road. |
| onIceRoad | On ice road | On ice road. |
| onLevelCrossing | On level crossing | On level-crossing. |
| onLinkRoad | On link road | On road section linking two different roads. |
| onRoundabout | On roundabout | On roundabout. |
| onTheRoadway | On the roadway | On the roadway. |
| onUndergroundSection | On underground section | On underground section of road. |
| onUnderpass | On underpass | On underpass, i.e. section of road which passes under another road. |
| withinJunction | Within junction | On the main carriageway within a junction between exit slip road and entry slip road. |

The <<D2Enumeration>> "LaneEnum"

List of descriptors identifying specific lanes.

Table A.106— Values contained in the enumeration "LaneEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| allLanesCompleteCarriageway | All lanes complete carriageway | In all lanes of the carriageway. |
| busLane | Bus lane | In the bus lane. |
| busStop | Bus stop | In the bus stop lane. |
| carPoolLane | Car pool lane | In the carpool lane. |
| centralReservation | Central reservation | On the central reservation separating the two directional carriageways of the highway. |
| crawlerLane | Crawler lane | In the crawler lane - a lane that should be used by slower vehicles. |
| cycleLane | Cycle lane | Part of a carriageway designated for cycles. A cycle lane is distinguished from the rest of the carriageway by longitudinal road markings. |
| emergencyLane | Emergency lane | In the emergency lane. |
| escapeLane | Escape lane | In the escape lane. |
| expressLane | Express lane | In the express lane. |
| hardShoulder | Hard shoulder | On the hard shoulder. |
| heavyVehicleLane | Heavy vehicle lane | In the heavy vehicle lane. |
| layBy | Lay by | In a lay-by. |
| leftHandTurningLane | Left hand turning lane | In the left hand turning lane. |
| leftLane | Left lane | In the left lane. |
| localTrafficLane | Local traffic lane | In the local traffic lane. |
| middleLane | Middle lane | In the middle lane. |
| overtakingLane | Overtaking lane | In the overtaking lane. |
| rightHandTurningLane | Right hand turning lane | In the right hand turning lane. |
| rightLane | Right lane | In the right lane. |
| rushHourLane | Rush hour lane | In the lane dedicated for use during the rush (peak) hour. |
| setDownArea | Set down area | In the area/lane reserved for passenger pick-up or set-down. |
| slowVehicleLane | Slow vehicle lane | In a lane dedicated to vehicles that are not permitted to exceed a fixed slow speed. |
| throughTrafficLane | Through traffic lane | In the through traffic lane. |
| tidalFlowLane | Tidal flow lane | In the lane dedicated for use as a tidal flow lane. |
| turningLane | Turning lane | In the turning lane. |
| verge | Verge | On the verge. |

The <<D2Enumeration>> "LinearDirectionEnum"

Directions of traffic flow relative to the direction in which the linear element is defined.

Table A.107— Values contained in the enumeration "LinearDirectionEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| aligned | Aligned | Indicates that the direction of traffic flow affected by the situation or related to the traffic data is in the same sense as the direction in which the linear element is defined. |
| both | Both | Indicates that both directions of traffic flow are affected by the situation or relate to the traffic data. |
| opposite | Opposite | Indicates that the direction of traffic flow affected by the situation or related to the traffic data is in the opposite sense to the direction in which the linear element is defined. |
| unknown | Unknown | Indicates that the direction of traffic flow affected by the situation or related to the traffic data is unknown. |

The <<D2Enumeration>> "LinearElementNatureEnum"

List of indicative natures of linear elements.

Table A.108— Values contained in the enumeration "LinearElementNatureEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| other | Other | Other than as defined in this enumeration. |
| road | Road | The nature of the linear element is a road. |
| roadSection | Road section | The nature of the linear element is a section of a road. |
| slipRoad | Slip road | The nature of the linear element is a slip road. |

The <<D2Enumeration>> "MaintenanceVehicleActionsEnum"

Types of maintenance vehicle actions associated with roadworks.

Table A.109— Values contained in the enumeration "MaintenanceVehicleActionsEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| slowMoving | Slow moving | Maintenance vehicles are slow moving. |

The <<D2Enumeration>> "NamedAreaTypeEnum"

Types of areas.

Table A.110— Values contained in the enumeration "NamedAreaTypeEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| applicationRegion | Application region | Application region |
| carParkArea | Car park area | Car park area |
| carpoolArea | Carpool area | Carpool area |
| continent | Continent | Continent |
| country | Country | Country |
| countryGroup | Country group | Group of countries. |
| fuzzyArea | Fuzzy area | Fuzzy area |
| industrialArea | Industrial area | Industrial area |
| lake | Lake | Lake |
| meteorologicalArea | Meteorological area | Meteorological area |
| metropolitanArea | Metropolitan area | Metropolitan area |
| municipality | Municipality | Municipality |
| order1AdministrativeArea | Order1 administrative area | Order 1 administrative area |
| order2AdministrativeArea | Order2 administrative area | Order 2 administrative area |
| order3AdministrativeArea | Order3 administrative area | Order 3 administrative area |
| order4AdministrativeArea | Order4 administrative area | Order 4 administrative area |
| order5AdministrativeArea | Order5 administrative area | Order 5 administrative area |
| parkAndRideSite | Park and ride site | A park and ride site |
| policeForceControlArea | Police force control area | Police force control area |
| roadOperatorControlArea | Road operator control area | Road operator control area |
| ruralCounty | Rural county | Rural county |
| sea | Sea | Sea |
| touristArea | Tourist area | Tourist area |
| trafficArea | Traffic area | Traffic area |
| urbanCounty | Urban county | Urban county |
| waterArea | Water area | Water area |

The <<D2Enumeration>> "NonWeatherRelatedRoadConditionTypeEnum"

Types of road surface conditions which are not related to the weather.

Table A.111— Values contained in the enumeration "NonWeatherRelatedRoadConditionTypeEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| looseChippings | Loose chippings | Increased skid risk and injury risk due to loose chippings on road. |
| mudOnRoad | Mud on road | Increased skid risk due to mud on road. |
| oilOnRoad | Oil on road | Increased skid risk due to oil on road. |
| petrolOnRoad | Petrol on road | Increased skid risk due to petrol on road. |
| slipperyRoad | Slippery road | The road surface is slippery due to an unspecified non-weather related cause. |

The <<D2Enumeration>> "NutsCodeTypeEnum"

Types of NUTS codes (Nomenclature of territorial units for statistics) including LAU codes (Local Administrative Units).

Table A.112— Values contained in the enumeration "NutsCodeTypeEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| lau1Code | Lau1 code | LAU 1 code |
| lau2Code | Lau2 code | LAU 2 code |
| nuts1Code | Nuts1 code | NUTS 1 code |
| nuts2Code | Nuts2 code | NUTS 2 code |
| nuts3Code | Nuts3 code | NUTS 3 code |

The <<D2Enumeration>> "ObstructionTypeEnum"

Types of obstructions on the roadway.

Table A.113— Values contained in the enumeration "ObstructionTypeEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| childrenOnRoadway | Children on roadway | Children on the roadway which may cause traffic disruption. |
| clearanceWork | Clearance work | Clearance work associated with an earlier traffic problem which may cause traffic disruption. |
| cyclistsOnRoadway | Cyclists on roadway | Cyclists on the roadway which may cause traffic disruption. |
| incident | Incident | Incidents are chance occurrences involving vehicles from the traffic stream, which could present potential hazards to road users. This item excludes accidents. |
| objectOnTheRoad | Object on the road | The road may be obstructed or traffic hindered due to objects laying on the roadway. |
| peopleOnRoadway | People on roadway | People on the roadway which may cause traffic disruption. |
| rescueAndRecoveryWork | Rescue and recovery work | Work is being undertaken by emergency services which may present a hazard to road users. |
| shedLoad | Shed load | Spillage of transported goods on the roadway which may cause traffic disruption. |
| unprotectedAccidentArea | Unprotected accident area | An accident area which has not been protected and may present a hazard to road users. |

The <<D2Enumeration>> "OpenlrFormOfWayEnum"

Enumeration of for of way

Table A.114— Values contained in the enumeration "OpenlrFormOfWayEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| motorway | Motorway | Motorway |
| multipleCarriageway | Multiple carriageway | Multiple carriageway |
| other | Other | Other |
| roundabout | Roundabout | Roundabout |
| singleCarriageway | Single carriageway | Single carriageway |
| slipRoad | Slip road | Slip road |
| trafficSquare | Traffic square | Traffic square |
| undefined | Undefined | Undefined |

The <<D2Enumeration>> "OpenlrFunctionalRoadClassEnum"

Enemuration of functional road class

Table A.115— Values contained in the enumeration "OpenlrFunctionalRoadClassEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| frc0 | Frc0 | Main road, highest importance |
| frc1 | Frc1 | First class road |
| frc2 | Frc2 | Second class road |
| frc3 | Frc3 | Third class road |
| frc4 | Frc4 | Fourth class road |
| frc5 | Frc5 | Fifth class road |
| frc6 | Frc6 | Sixth class road |
| frc7 | Frc7 | Other class road, lowest importance |

The <<D2Enumeration>> "OpenlrOrientationEnum"

Enumeration of side of road

Table A.116— Values contained in the enumeration "OpenlrOrientationEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| againstLineDirection | Against line direction | Against line direction |
| both | Both | Both directions |
| noOrientationOrUnknown | No orientation or unknown | No orientation or unknown |
| withLineDirection | With line direction | With line direction |

The <<D2Enumeration>> "OpenlrSideOfRoadEnum"

Enumeration of side of road

Table A.117— Values contained in the enumeration "OpenlrSideOfRoadEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| both | Both | On both sides of the road. |
| left | Left | On the left side of the road. |
| onRoadOrUnknown | On road or unknown | On road or unknown |
| right | Right | On the right side of the road. |

The <<D2Enumeration>> "PositionConfidenceCodedErrorEnum"

Error code for horizontal or vertical position confidence

Table A.118— Values contained in the enumeration "PositionConfidenceCodedErrorEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| outOfRange | Out of range | Indicates the accuracy is out of range, i.e. greater than 4 093 cm for horizontal position. |
| unavailable | Unavailable | Indicates the accuracy information is unavailable. |

The <<D2Enumeration>> "ProbabilityOfOccurrenceEnum"

Levels of confidence that the sender has in the information, ordered {certain, probable, risk of}.

Table A.119— Values contained in the enumeration "ProbabilityOfOccurrenceEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| certain | Certain | The source is completely certain of the occurrence of the situation record version content. |
| probable | Probable | The source has a reasonably high level of confidence of the occurrence of the situation record version content. |
| riskOf | Risk of | The source has a moderate level of confidence of the occurrence of the situation record version content. |

The <<D2Enumeration>> "ReferentTypeEnum"

A set of types of known points along a linear object such as a road.

Table A.120— Values contained in the enumeration "ReferentTypeEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| boundary | Boundary | A boundary between two jurisdictional or administrative areas. These may be legal boundaries such as between counties or countries, maintenance responsibility boundaries or control boundaries. |
| intersection | Intersection | A crossing of two or more roads where the precise point of intersection is defined according to specific business rules. |
| landmark | Landmark | A visible identifiable physical landmark either alongside or close to the linear object. |
| referenceMarker | Reference marker | A marker which is usually but not necessarily physical that is one of a sequence which are spaced out along the linear object (road) to provide a location reference. The spacing of markers is not necessarily even. |
| roadNode | Road node | A topological node defined on a road network. Such nodes may delineate the segmentation of the road network according to defined business rules or may constitute a purely topological representation of a road network. |

The <<D2Enumeration>> "RelativePositionOnCarriagewayEnum"

Identifies a relative position across a carriageway

Table A.121— Values contained in the enumeration "RelativePositionOnCarriagewayEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| inTheCentre | In the centre | In the centre of the roadway. |
| onTheLeft | On the left | On the left of the roadway. |
| onTheRight | On the right | On the right of the roadway. |

The <<D2Enumeration>> "RoadMaintenanceTypeEnum"

Types of road maintenance.

Table A.122— Values contained in the enumeration "RoadMaintenanceTypeEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| maintenanceWork | Maintenance work | Maintenance of road, associated infrastructure or equipments. |
| roadMarkingWork | Road marking work | Striping and repainting of road markings, plus placement or replacement of reflecting studs (cats' eyes). |

The <<D2Enumeration>> "SubdivisionTypeEnum"

ISO 3166-2 subdivison types.

Table A.123— Values contained in the enumeration "SubdivisionTypeEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| administrativeAtoll | Administrative atoll | Administrative atoll |
| administrativeRegion | Administrative region | Administrative region |
| administrativeTerritory | Administrative territory | Administrative territory |
| arcticRegion | Arctic region | Arctic region |
| autonomousCity | Autonomous city | Autonomous city |
| autonomousCityInNorthAfrica | Autonomous city in north africa | Autonomous city in North Africa |
| autonomousCommunity | Autonomous community | Autonomous community |
| autonomousDistrict | Autonomous district | Autonomous district |
| autonomousProvince | Autonomous province | Autonomous province |
| autonomousRegion | Autonomous region | Autonomous region |
| canton | Canton | Canton |
| capitalCity | Capital city | Capital city |
| city | City | City |
| cityMunicipality | City municipality | City municipality |
| cityOfCountyRight | City of county right | City of county right |
| commune | Commune | Commune |
| councilArea | Council area | Council area |
| country | Country | Country |
| county | County | County |
| department | Department | Department |
| dependency | Dependency | Dependency |
| district | District | District |
| districtMunicipality | District municipality | District municipality |
| districtWithSpecialStatus | District with special status | District with special status |
| entity | Entity | Entity |
| geographicalEntity | Geographical entity | Geographical entity |
| governorate | Governorate | Governorate |
| laender | Laender | Länder |
| localCouncil | Local council | Local Council |
| londonBorough | London borough | London borough |
| metropolitanArea | Metropolitan area | Metropolitan area |
| metropolitanDepartment | Metropolitan department | Metropolitan department |
| metropolitanDistrict | Metropolitan district | Metropolitan district |
| metropolitanRegion | Metropolitan region | Metropolitan region |
| municipality | Municipality | Municipality |
| other | Other | Other |
| overseasDepartment | Overseas department | Overseas department |
| overseasRegion | Overseas region | Overseas region |
| overseasTerritorialCollectivity | Overseas territorial collectivity | Overseas territorial collectivity |
| parish | Parish | Parish |
| province | Province | Province |
| quarter | Quarter | Quarter |
| region | Region | Region |
| republic | Republic | Republic |
| republicanCity | Republican city | Republic city |
| selfGovernedPart | Self governed part | Self-governed part |
| specialMunicipality | Special municipality | Special Municipality |
| state | State | State |
| territorialUnit | Territorial unit | Territorial unit |
| territory | Territory | Territory |
| twoTierCounty | Two tier county | Two tier country |
| unitaryAuthority | Unitary authority | Unitary Authority |
| ward | Ward | Ward |

The <<D2Enumeration>> "TpegLoc01AreaLocationSubtypeEnum"

Types of area.

Table A.124— Values contained in the enumeration "TpegLoc01AreaLocationSubtypeEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| largeArea | Large area | A geographic or geometric large area. |
| other | Other | Other than as defined in this enumeration. |

The <<D2Enumeration>> "TpegLoc01FramedPointLocationSubtypeEnum"

Types of points on the road network framed by two other points on the same road.

Table A.125— Values contained in the enumeration "TpegLoc01FramedPointLocationSubtypeEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| framedPoint | Framed point | A point on the road network framed by two other points on the same road. |

The <<D2Enumeration>> "TpegLoc01LinearLocationSubtypeEnum"

Types of linear location.

Table A.126— Values contained in the enumeration "TpegLoc01LinearLocationSubtypeEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| segment | Segment | A segment (or link) of the road network corresponding to the way in which the road operator has segmented the network. |

The <<D2Enumeration>> "TpegLoc01SimplePointLocationSubtypeEnum"

Types of simple point.

Table A.127— Values contained in the enumeration "TpegLoc01SimplePointLocationSubtypeEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| intersection | Intersection | An point on the road network at which one or more roads intersect. |
| nonLinkedPoint | Non linked point | A point on the road network which is not at a junction or intersection. |

The <<D2Enumeration>> "TpegLoc03AreaDescriptorSubtypeEnum"

Descriptors for describing area locations.

Table A.128— Values contained in the enumeration "TpegLoc03AreaDescriptorSubtypeEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| administrativeAreaName | Administrative area name | Name of an administrative area. |
| administrativeReferenceName | Administrative reference name | Reference name by which administrative area is known. |
| areaName | Area name | Name of an area. |
| countyName | County name | Name of a county (administrative sub-division). |
| lakeName | Lake name | Name of a lake. |
| nationName | Nation name | Name of a nation (e.g. Wales) which is a sub-division of a ISO recognised country. |
| other | Other | Other than as defined in this enumeration. |
| policeForceControlAreaName | Police force control area name | Name of a police force control area. |
| regionName | Region name | Name of a geographic region. |
| seaName | Sea name | Name of a sea. |
| townName | Town name | Name of a town. |

The <<D2Enumeration>> "TpegLoc03IlcPointDescriptorSubtypeEnum"

Descriptors for describing a junction by identifying the intersecting roads at a road junction.

Table A.129— Values contained in the enumeration "TpegLoc03IlcPointDescriptorSubtypeEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| tpegIlcName1 | TPEG ILC name1 | The name of the road on which the junction point is located. |
| tpegIlcName2 | TPEG ILC name2 | The name of the first intersecting road at the junction. |
| tpegIlcName3 | TPEG ILC name3 | The name of the second intersecting road (if one exists) at the junction. |

The <<D2Enumeration>> "TpegLoc03JunctionPointDescriptorSubtypeEnum"

Descriptors for describing a point at a road junction.

Table A.130— Values contained in the enumeration "TpegLoc03JunctionPointDescriptorSubtypeEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| junctionName | Junction name | Name of a road network junction where two or more roads join. |

The <<D2Enumeration>> "TpegLoc03OtherPointDescriptorSubtypeEnum"

Descriptors other than junction names and road descriptors which can help to identify the location of points on the road network.

Table A.131— Values contained in the enumeration "TpegLoc03OtherPointDescriptorSubtypeEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| administrativeAreaName | Administrative area name | Name of an administrative area. |
| administrativeReferenceName | Administrative reference name | Reference name by which an administrative area is known. |
| airportName | Airport name | Name of an airport. |
| areaName | Area name | Name of an area. |
| buildingName | Building name | Name of a building. |
| busStopIdentifier | Bus stop identifier | Identifier of a bus stop on the road network. |
| busStopName | Bus stop name | Name of a bus stop on the road network. |
| canalName | Canal name | Name of a canal. |
| countyName | County name | Name of a county (administrative sub-division). |
| ferryPortName | Ferry port name | Name of a ferry port. |
| intersectionName | Intersection name | Name of a road network intersection. |
| lakeName | Lake name | Name of a lake. |
| linkName | Link name | Name of a road link. |
| localLinkName | Local link name | Local name of a road link. |
| metroStationName | Metro station name | Name of a metro/underground station. |
| nationName | Nation name | Name of a nation (e.g. Wales) which is a sub-division of a ISO recognised country. |
| nonLinkedPointName | Non linked point name | Name of a point on the road network which is not at a junction or intersection. |
| other | Other | Other than as defined in this enumeration. |
| parkingFacilityName | Parking facility name | Name of a parking facility. |
| pointName | Point name | Name of a specific point. |
| pointOfInterestName | Point of interest name | Name of a general point of interest. |
| railwayStation | Railway station | Name of a railway station. |
| regionName | Region name | Name of a geographic region. |
| riverName | River name | Name of a river. |
| seaName | Sea name | Name of a sea. |
| serviceAreaName | Service area name | Name of a service area on a road network. |
| tidalRiverName | Tidal river name | Name of a river which is of a tidal nature. |
| townName | Town name | Name of a town. |

The <<D2Enumeration>> "TpegLoc04HeightTypeEnum"

Types of height.

Table A.132— Values contained in the enumeration "TpegLoc04HeightTypeEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| above | Above | Height above specified location. |
| aboveSeaLevel | Above sea level | Height above mean sea high water level. |
| aboveStreetLevel | Above street level | Height above street level. |
| at | At | At height of specified location. |
| atSeaLevel | At sea level | At mean sea high water level. |
| atStreetLevel | At street level | At street level. |
| below | Below | Height below specified location. |
| belowSeaLevel | Below sea level | Height below mean sea high water level. |
| belowStreetLevel | Below street level | Height below street level. |
| other | Other | Other than as defined in this enumeration. |
| undefined | Undefined | Undefined height reference. |
| unknown | Unknown | Unknown height reference. |

The <<D2Enumeration>> "TrafficConstrictionTypeEnum"

Types of constriction to which traffic is subjected as a result of an event.

Table A.133— Values contained in the enumeration "TrafficConstrictionTypeEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| carriagewayBlocked | Carriageway blocked | The carriageway is totally obstructed in the specified direction due to an unplanned event. |
| carriagewayPartiallyObstructed | Carriageway partially obstructed | The carriageway is partially obstructed in the specified direction due to an unplanned event. |
| lanesBlocked | Lanes blocked | One or more lanes is totally obstructed in the specified direction due to an unplanned event. |
| lanesPartiallyObstructed | Lanes partially obstructed | One or more lanes is partially obstructed in the specified direction due to an unplanned event. |
| roadBlocked | Road blocked | The road is totally obstructed, for all vehicles in both directions, due to an unplanned event. |
| roadPartiallyObstructed | Road partially obstructed | The road is partially obstructed in both directions due to an unplanned event. |

The <<D2Enumeration>> "ValidityStatusEnum"

Values of validity status that can be assigned to a described event, action or item.

Table A.134— Values contained in the enumeration "ValidityStatusEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| active | Active | The described event, action or item is currently active regardless of the definition of the validity time specification. |
| definedByValidityTimeSpec | Defined by validity time spec | The validity status of the described event, action or item is in accordance with the definition of the validity time specification. |
| planned | Planned | The described event, action or item is currently planned regardless of the definition of the validity time specification. |
| suspended | Suspended | The described event, action or item is currently suspended, that is inactive, regardless of the definition of the validity time specification. |

The <<D2Enumeration>> "VehicleObstructionTypeEnum"

Types of obstructions involving vehicles.

Table A.135— Values contained in the enumeration "VehicleObstructionTypeEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| brokenDownVehicle | Broken down vehicle | Broken down vehicle(s) on the carriageway which may cause traffic disruption. |
| vehicleOnFire | Vehicle on fire | A vehicle is or has been on fire and may cause traffic disruption. |
| vehicleOnWrongCarriageway | Vehicle on wrong carriageway | A vehicle is travelling the wrong way along a divided highway (i.e. on the wrong side). |

The <<D2Enumeration>> "VehicleStatusEnum"

The status of a vehicle.

Table A.136— Values contained in the enumeration "VehicleStatusEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| brokenDown | Broken down | Broken down vehicle (i.e. it is immobile due to mechanical breakdown). |
| burntOut | Burnt out | Burnt out vehicle, but fire is extinguished. |
| damaged | Damaged | Vehicle is damaged following an incident or collision. It may be able or not to move by itself. |
| damagedAndImmobilized | Damaged and immobilized | Vehicle is damaged following an incident or collision. It is immobilized and therefore needs assistance to be moved. |
| jacknifed | Jacknifed | The pulling vehicle is in a jackknifed position with its trailer |
| onFire | On fire | Vehicle is on fire. |
| overturned | Overturned | Vehicle is on its side or upside down |
| spunAround | Spun around | Vehicle has come to rest not facing its intended line of travel. |

The <<D2Enumeration>> "VehicleTypeEnum"

Types of vehicle.

Table A.137— Values contained in the enumeration "VehicleTypeEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| bicycle | Bicycle | Bicycle. |
| bus | Bus | Bus. |
| longHeavyLorry | Long heavy lorry | A heavy lorry that is longer than normal. |
| lorry | Lorry | Lorry of any type. |
| moped | Moped | Moped (a two wheeled motor vehicle characterized by a small engine typically less than 50cc and by normally having pedals). |
| motorcycle | Motorcycle | Motorcycle. |

The <<D2Enumeration>> "WeatherRelatedRoadConditionTypeEnum"

Types of road surface conditions which are related to the weather.

Table A.138— Values contained in the enumeration "WeatherRelatedRoadConditionTypeEnum"

| Enumerated value name | Designation | Definition |
| --- | --- | --- |
| blackIce | Black ice | Severe skid risk due to black ice (i.e. clear ice, which is impossible or very difficult to see). |
| ice | Ice | Increased skid risk due to ice (of any kind). |
| icyPatches | Icy patches | Severe skid risk due to icy patches (i.e. intermittent ice on roadway). |
| snowDrifts | Snow drifts | Snow drifting is in progress or patches of deep snow are present due to earlier drifting. |
| surfaceWater | Surface water | Water is resting on the roadway which provides an increased hazard to vehicles. |